|  | CHARLOTTE COUNTY COMPREHENSIVE VULNERABILITY ASSESSMENT |
| --- | --- |
|  | Client: Charlotte County, FL Funded By: COASTAL AND HEARTLAND NATIONAL ESTUARY PARTNERSHIP (CHNEP) |
|  | ADAPTATION Focus AREA and RECOMMENDATION Report |
|  | DATE: May 2026 |
|  | WSP USA |

Table of contents

1	Overview	1

2	Objectives	1

3	Methodology	2

3.1	Data-Driven Analysis	2

3.2	Selection Criteria	2

3.3	Facility Ranking Basis 	2

3.3.1	Flood-induced monetized losses	3

3.3.2	Flood damage severity	3

4	Identified Adaptation Focus Areas	4

4.1	Adaptation Focus Area 1 – South	11

4.1.1	Focus Area Overview	11

4.1.2	Impacted Facilities	11

4.2	Adaptation Focus Area 2 – Mid	31

4.2.1	Focus Area Overview	31

4.2.2	Impacted Facilities	31

4.3	Adpataion Focus Area 3 – West	65

4.3.1	Focus Area Overview	65

4.3.2	Impacted Facilities	65

5	Next Steps	88

5.1	Alignment with Charlotte County Local Mitigation Strategy	88

5.2	Recommended Actions to Increase Resilience of Charlotte County	89

5.2.1	Countywide Planning and Policy	89

5.2.2	Wastewater Treatment Facility	90

5.2.3	Emergency Response Facilities (Fire Stations)	90

5.2.4	Healthcare Facilities and Hospitals	90

5.2.5	Schools and Shelter Facilities	90

5.2.6	Community Facilities	91

5.2.7	Local Government and Administrative Facilities	91

5.2.8	Parks	91

5.2.9	Transportation Infrastructure	91

5.2.10	Homeowner Flood Resilience Actions	91

5.3	Industry Standards Cost Estimation	92

5.4	Potential Funding Sources	92

5.4.1	Potential Federal and State Funding Sources	93

5.4.2	Alternative Funding Sources	95

6	Summary	96

List of Figures

Figure 1 Charlotte County Adaptation Focus Areas	5

Figure 2 Charlotte County Adaptation Focus Areas: Current Storm Surge Flooding	6

Figure 3 Charlotte County Adaptation Focus Areas: 2040 Intermediate-High Storm Surge Flooding plus 1ft Sea Level Change	7

Figure 4 Charlotte County Adaptation Focus Areas: 2070 Intermediate-High Storm Surge Flooding plus 3ft Sea Level Change	8

List of Tables

Table 1 Flood depth threshold for various flood damage severities	3

Table 2 Summary of the Critical Infrastructure Buildings Present in Each Adaptation Focus Area	8

Table 3 Summary of the Critical Community and Emergency Facilities Buildings Present in Each Adaptation Focus Area	9

Table 4 Summary of Transportation and Natural Resources Present in Each Adaptation Focus Area	10

Table 5 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 1- South under Current Condition	12

Table 6 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 1- South when Change in Sea Level Exceeds 1 ft	16

Table 7 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 1- South when Change in Sea Level Exceeds 3 ft	21

Table 8 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 2 - Mid under Current Condition	32

Table 9 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 2 - Mid when Change in Sea Level Exceeds 1 ft	37

Table 10 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 2 - Mid when Change in Sea Level Exceeds 3 ft	45

Table 11 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 3 - West under Current Condition	66

Table 12 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 3 - West when Change in Sea Level Exceeds 1 ft	72

Table 13 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 3 - West when Change in Sea Level Exceeds 3 ft	78

Table 14 Alignment between Identified Vulnerable Facilities and LMS Projects	88

Table 15 Industry Standard Costs Reference	92

Table 16 Potential Federal and State Funding Sources Funding Sources	93

# Overview

Charlotte County faces significant and growing exposure to multiple flood hazards, including storm surge from tropical systems, extreme rainfall, king tide flooding, and long‑term sea level change. The County is currently completing a Vulnerability Assessment (VA) designed to meet Florida Department of Environmental Protection (FDEP) standards, which evaluates how critical community assets, such as government facilities, public safety buildings, water/wastewater plants, energy infrastructure, and transportation routes, may be impacted by future flood conditions. The VA incorporates the best available modeling of tidal flooding, storm surge, and sea level change to determine which assets and geographic areas face the greatest risk.

The County’s flood analysis highlights that Charlotte County is a low‑lying coastal region influenced by the Peace and Myakka Rivers, Charlotte Harbor, the Gulf of America, and an extensive network of tributaries. Flooding may result from upstream river rise, high tides, coastal surge, and compound events involving multiple water sources. These combined hazards create vulnerability across coastal and riverine communities. In this report, the storm surge and sea level change are considered as the main flooding stressor to meet the requirements for a compliant VA under FDEP guidance.

The Adaptation Focus Area determination is essential to align the county’s long-term resilience strategy with FDEP requirements. Adaptation Action Areas are an area of policy focus within FDEP guidance, and the county will be transitioning to that in the future.  This focus area determination within that program allows the county to establish areas of initial policy concern and identify them within this VA documentation.

Many grant and funding programs typically require references to past planning studies, including confirmation that affected assets are located within designated areas of concern for county policy.  The county has established the policy areas noted in this document as geographic locations where the combination of potential flooding and community impacts is of such concern that they need to be designated for action.

It is worth noting that this assessment currently focuses on county-owned assets (as per FDEP guidance) and does not encompass all county concerns, such as economic or community-wide needs. This limitation results in a decision-making gap that could restrict the scope of resilience initiatives if left unaddressed. To partially help to address this, it was identified that the effort to establish these areas should follow FDEP guidance on requirements and be as inclusive as possible.

The ultimate goal of identifying the Adaptation Focus Areas is to guide Charlotte County’s future adaptation strategies, allowing the County to prioritize resilience actions, optimize resource allocation, and strengthen emergency preparedness for flood‑prone areas.

# Objectives

The primary objectives of this Adaptation Focus Areas identification are to:

- Prioritize Vulnerable Locations: Identify geographical areas with the highest concentration of vulnerable critical assets based on quantitative exposure and sensitivity analysis results.

- Guide Adaptation Planning: Establish focused areas for developing targeted adaptation strategies that maximize the protection of critical community functions and services.

- Optimize Resource Allocation: Enable the County to strategically direct limited resources toward areas with the greatest vulnerability and adaptation potential.

- Enhance Community Resilience: Focus adaptation efforts on locations that are essential for maintaining community functionality during and after flood events.

- Support Emergency Planning: Identify critical areas requiring enhanced emergency preparedness and response capabilities.

- Comply with FDEP Guideline: Ensure Adaptation Action Areas Focus Areas identification follows Resilient Florida Program guidelines and GIS data standards for consistency with statewide planning efforts.

# Methodology

The methodology for identifying Adaptation Focus Areas for Charlotte County follows the process outlined in Chapter 2 of the 2018 Florida Adaptation Planning Guidebook. It is a hybrid approach that combines robust, data-driven analysis with client engagement to ensure the selected areas are both technically defensible and reflective of community priorities. This methodology is designed to ensure that the identification of Adaptation Focus Areas is not only grounded in the best available scientific data but also informed by the local knowledge and priorities of the community.

## Data-Driven Analysis

The core of the methodology involved a geospatial analysis that overlaid the results of the flood hazard modeling with the County’s critical asset inventory. This allowed for the identification of geographic “hot spots” where risk is concentrated and where the potential for cascading failures is highest.

Key data inputs included:

- Critical Assets Inventory: A comprehensive GIS database of the County’s critical assets, categorized as required by the Florida Statute 380.093 into Critical Community and Emergency Facilities; Critical Infrastructure; Transportation and Evacuation Routes; and Natural, Cultural, and Historical Resources.

- Exposure and Sensitivity Analysis Results: Detailed findings from the previous project tasks, which quantified the number of assets exposed to flooding and their degree of sensitivity under different flood scenarios. This data was crucial for identifying which assets are most at risk and under which conditions.

- Flood Hazard Models: Storm surge and tidal flooding models for current and future conditions with sea level change projections.

- Transportation Networks: Major roadways, evacuation routes, and critical transportation infrastructure.

## Selection Criteria

Based on the 2018 Florida Adaptation Planning Guidebook and best practices from other Florida communities, the following criteria were used to guide the selection and delineation of Adaptation Focus Areas:

- Concentration of Vulnerable Assets: Areas with a high spatial density of critical assets determined to be sensitive to flood hazards.

- Presence of Regionally Significant or Sole-Function Assets: Areas containing critical assets whose failure would have cascading, county-wide, or regional consequences (e.g., the wastewater treatment facility).

- Flood Hazard Risks: Areas where multiple flood hazards are likely to interact, creating a greater total impact.

- Community and Economic Significance: Areas vital to the County’s economy, cultural identity, and daily function.

- Historical Flooding Experience: Areas with recurring flooding problems based on local knowledge and past storm events that validate technical modeling results.

- Critical Service Dependencies: Areas where interdependencies between critical assets could create cascading failures affecting multiple neighborhoods or essential services.

## Facility Ranking Basis

The facility ranking process undertaken in this effort serves to inform the identification and justification of Adaptation Focus Areas. By translating analytical results into a structured, planning-level framework, this approach synthesizes data on flood exposure, damage potential, and estimated losses to pinpoint geographic locations where flood hazards intersect with essential services. The primary objective is to establish a clear rationale for directing resilience planning initiatives towards specific areas of Charlotte County that are most vulnerable to flood-related impacts.

This ranking is grounded in anticipated flood-induced monetized losses, which have been estimated in risk assessment (details can be found in the exposure and vulnerability report). Meanwhile, the flood damage severity is provided utilizing damage classification concepts (affected, minor, major, and destroyed) adapted from FEMA’s Preliminary Damage Assessment guidance to ensure a consistent screening process. The definition of these two primary factors is provided below:

- Flood-induced monetized losses: Cost to the county from flood-related impacts for a facility, including direct damage, repair costs, loss of service, and downtime-related indirect losses. These losses may oCCUr even when direct structural damage is limited or absent, particularly where surrounding flooding affects access, operations, inspection, cleanup, or restoration time.

- Flood damage severity: Direct physical damage to a facility from flood depth relative to the first floor elevation – in terms of loss percentage, to outline how complete the impacts may be to individual facilities, regardless of the value of the building itself, which was a similarly important policy point.

### Flood-induced monetized losses

Flood-induced monetized loss levels were classified into four categories to support relative ranking and screening of impacted facilities. Facilities with estimated losses greater than $15 million were classified as High, losses between $10 million and $15 million were classified as Medium, losses between $5 million and $10 million were classified as Low, and losses greater than $0 but less than $5 million were classified as Minimal. Facilities with no estimated losses were not assigned a monetized loss level. These thresholds are intended to distinguish relative levels of financial impact across facilities and support prioritization, rather than to represent absolute damage thresholds.

### Flood damage severity

To identify values to determine a method for estimating some categories of loss to reflect the flood damage severity, specifically relating to the % of loss for flood depth.  To develop the threshold for each flood damage severity based on the available data applied, the team:

- Assembled data points for the percentage of damage to a building for contents, and the structure itself (maintained as two different data points by FEMA)

- Combined the data from step 1 into a single defined value (HAZUS Inventory Technical Manual Tables 6-2 and 6-9).

- Determined the percentage loss by flood depth for all facility types included in the county analysis (see graphic below).

- Identified the various categories that roughly represented damage percentages to represent four levels: affected (10%), Minor (20%), Major (up to 50%), and Destroyed (above 50%)

- Calculated the flood depth that represented the four levels of impact noted in #4 above for each facility type

- Calculated the average flood depth that best represented that range across all facility types

The results of that analysis are shown in the table provided (Table 1).

Table 1 Flood depth threshold for various flood damage severities

| Damage Severity | Damage state | Flood depth | Damage ratio relative to replacement cost |
| --- | --- | --- | --- |
| Affected | Facilities with minimal cosmetic damage (e.g., water depth below the first floor elevation with limited damage to skirting or enclosure elements) and no disruption to essential operations. | 0-10’’ | 0-10% |
| Minor | Facilities with non-structural damage (e.g., water depth has reached the first floor elevation but limited in oCCUpied, operational, or equipment spaces). | 10’’-25’’ | 10%-20% |
| Major | Facilities sustaining structural or functional damage requiring extensive repairs (e.g., water depth within oCCUpied or operational spaces but below the ceiling), with likely interruption of essential services. | 25’’-68’’ | 20%-50% |
| Destroyed | Facilities are a high loss (e.g., structural frame is bent, twisted, or otherwise compromised), requiring replacement to restore service. | >68’’ | >50% |

It should be noted that the flood damage severity classification is intended to represent direct physical or functional damage to the facility, as informed by FEMA/HAZUS depth-damage relationships. As such, the damage severity categories are applied only when the modeled flood depth reaches or exceeds the first-floor elevation. Facilities with negative flood depths are therefore not assigned a damage severity category, because the modeled floodwater does not directly inundate the oCCUpied or damageable building space. Nevertheless, these facilities may still incur flood-induced monetized losses. The loss estimation method incorporates restoration-time assumptions that assign limited downtime for flood depths between -4 ft and 0 ft. Losses in this range reflect indirect operational impacts, including potential access constraints, temporary service disruption, cleanup or inspection needs, and delayed restoration. Accordingly, negative-depth facilities may appear with no assigned damage severity category while still being included in the loss ranking as Minimal losses.

By adopting this loss-based screening approach with damage severity information, the process ensures a consistent and defensible comparison of facilities throughout the county, thereby identifying geographic areas where flood risks pose the greatest threat to critical services and infrastructure.

# Identified Adaptation Focus Areas

Based on the application of the methodology and criteria described above, three Adaptation Focus Areas have been identified for Charlotte County, as shown in Figure 1. The following sections provide a detailed description of each Adaptation Focus Area, including the rationale for its selection, a comprehensive summary of the critical assets located within its boundaries, and maps showing its geographic extent and exposure to flood hazards (Figure 2).

Figure 1 Charlotte County Adaptation Focus Areas

Figure 2 Charlotte County Adaptation Focus Areas: Current Storm Surge Flooding

Figure 3 Charlotte County Adaptation Focus Areas: 2040 Intermediate-High Storm Surge Flooding plus 1ft Sea Level Change

Figure 4 Charlotte County Adaptation Focus Areas: 2070 Intermediate-High Storm Surge Flooding plus 3ft Sea Level Change

Table 2 Summary of the Critical Infrastructure Buildings Present in Each Adaptation Focus Area

| Critical Infrastructure | Total Quantity | South | Mid | West | Not included |
| --- | --- | --- | --- | --- | --- |
| Wastewater Treatment Facilities | 55 | 17 | 8 | 30 | 0 |
| Solid And Hazardous Waste Facilities | 9 | 0 | 2 | 1 | 6 |
| Communications Facilities | 6 | 2 | 3 | 1 | 0 |
| Pump Stations | 18 | 0 | 6 | 12 | 0 |
| Lift Station Facilities | 2 | 0 | 2 | 0 | 0 |
| Total | 90 | 19 | 21 | 44 | 6 |

Table 2 provides a breakdown of critical infrastructure facilities across three areas: south, mid, and west, as well as those not included in these regions. The categories listed are Wastewater Treatment Facilities, Solid and Hazardous Waste Facilities, Communications Facilities, Pump Stations, and Lift Station Facilities. Wastewater Treatment Facilities represent the largest group, with a total of 55 facility buildings. Of these, 17 are in Area south, 8 in Area mid, and 30 in Area west, with none excluded from the focus areas. Solid and Hazardous Waste Facilities total 9, with 2 in Area mid and 1 in Area west, while the remaining 6 are not included in any focus area due to their geolocation. Specifically, these 6 facilities are located farther inland, away from any identified adaptation focus areas, and are not indicated to experience tidal surge risk. As a result, they are not considered at significant risk, since their locations are not susceptible to tidal surge or similar coastal hazards. Communications Facilities are fewer, totaling 6, distributed as 2 in Area south, 3 in Area mid, and 1 in Area west. Pump Stations account for 18 facilities, all located in Area mid (6) and Area west (12), with none in Area south or outside the focus areas. Lift Station Facilities are the least numerous, with just 2, both situated in Area mid. The total count for all facility buildings is 90, with 19 in Area south, 21 in Area mid, 44 in Area west, and 6 not included in any Adaptation focus area. This distribution highlights that Area west contains the largest share of critical infrastructure, particularly in wastewater treatment and pump stations, while Area south and Area mid have fewer facilities overall.

Table 3 Summary of the Critical Community and Emergency Facilities Buildings Present in Each Adaptation Focus Area

| Critical Community and Emergency Facilities | Total Quantity | South | Mid | West | Not included |
| --- | --- | --- | --- | --- | --- |
| Fire Stations | 23 | 7 | 7 | 8 | 1 |
| Correctional Facilities | 28 | 6 | 0 | 0 | 22 |
| Local Government Facilities | 22 | 11 | 9 | 2 | 0 |
| Logistical Staging Areas | 1 | 1 | 0 | 0 | 0 |
| Health Care Facilities | 21 | 5 | 13 | 3 | 0 |
| Schools | 108 | 15 | 74 | 19 | 0 |
| Logistical Staging Areas, Emergency Operation Centers, Correctional Facilities | 1 | 1 | 0 | 0 | 0 |
| Emergency Medical Service Facilities | 2 | 1 | 1 | 0 | 0 |
| Affordable Public Housing | 7 | 2 | 5 | 0 | 0 |
| Law Enforcement Facilities, Airports | 1 | 1 | 0 | 0 | 0 |
| Law Enforcement Facilities | 3 | 2 | 1 | 0 | 0 |
| Airports, Law Enforcement Facilities | 1 | 1 | 0 | 0 | 0 |
| Colleges And Universities | 9 | 9 | 0 | 0 | 0 |
| Airports, Fire Stations | 1 | 1 | 0 | 0 | 0 |
| Community Centers | 11 | 1 | 8 | 2 | 0 |
| Law Enforcement Facilities, Fire Stations | 1 | 1 | 0 | 0 | 0 |
| Disaster Recovery Centers, Community Centers | 2 | 0 | 1 | 1 | 0 |
| Hospitals | 8 | 1 | 7 | 0 | 0 |
| Schools, Risk Shelter Inventory | 6 | 0 | 6 | 0 | 0 |
| Total | 256 | 66 | 132 | 35 | 23 |

Table 3 presents a detailed inventory of critical community and emergency facilities across the three Adaptation focus areas. In total, 256 facilities are catalogued. Area mid houses the largest number of facility buildings, with 132 in total, followed by Area south (66), Area west (35), and 23 facility buildings outside the designated focus areas. Schools make up the most substantial category, numbering 108, with the majority concentrated in Area mid (74), reflecting its role as an educational hub. Fire stations are relatively evenly distributed, with 7 each in Area south and mid, 8 in west, and one outside the focus areas, indicating broad coverage across the regions for fire protection services. Correctional facilities are notable for their presence largely outside the Adaptation focus areas, with 22 of the 28 total facility buildings situated beyond the defined zones. Local government facilities, on the other hand, are primarily located within Area south (11) and Area mid (9), suggesting these areas serve as administrative centres. Health care facilities are also heavily concentrated in Area mid (13 of 21), which includes hospitals (7 of 8), emphasizing the importance of this region in providing medical care and support. Logistical and emergency operation centres, affordable public housing, community centres, and law enforcement facilities are distributed across the regions, with Area mid and south containing the majority. Area west, while having fewer facilities overall, maintains a mix of schools, fire stations, and community centres, contributing to its infrastructure for emergency response and community support. The presence of specialized assets such as airports, colleges and universities, and disaster recovery centres further illustrates the multifaceted nature of the regions' preparedness and service provision. For example, Area south contains all nine colleges and universities, while Area mid is the primary location for risk shelter inventory associated with schools.

Table 4 Summary of Transportation and Natural Resources Present in Each Adaptation Focus Area

| Adaptation Focus Area | Total Miles |  | Count |  |  | Total Acres |
| --- | --- | --- | --- | --- | --- | --- |
|  | Major Roads | Rail | Standalone Lift Station | Bridges/Culverts | Historic Bridges | Parks |
| 1 - South | 131.2 | 16.2 | 54 | 26 | 3 | 30,078.0 |
| 2 - Mid | 176.9 | 0 | 194 | 68 | 0 | 374.9 |
| 3 - West | 106.2 | 0 | 104 | 56 | 6 | 577.6 |
| Outside | 67.8 | 8.8 | 10 | 2 | 0 | 28.3 |

Table 4 provides a comparative overview of various infrastructure and community assets across the three Adaptation Focus Areas and those located outside the designated focus areas. The categories examined include major roads, rail, standalone lift stations, bridges/culverts, historic bridges, and parks, with the respective measurements in miles, counts, and total acres.

- Adaptation Focus Area 1 – South stands out with the second highest mileage of major roads (131.2 miles) and a significant presence of rail infrastructure (16.2 miles). It also has a moderate count of standalone lift stations (54), bridges/culverts (26), and historic bridges (3). Notably, the South area contains an exceptionally large share of parks, totaling 30,078 acres, which far exceeds the park acreage in other areas.

- Adaptation Focus Area 2 – Mid has the highest mileage of major roads (176.9 miles), suggesting it is a central corridor for transportation. Although it lacks rail infrastructure, it leads in the number of  standalone lift stations (194) and bridges/culverts (68), which may point to a dense development pattern and the need for extensive stormwater management. However, its park acreage is relatively modest at 374.9 acres, possibly due to more urbanized land use or limited open space compared to the South.

- Adaptation Focus Area 3 – West features a smaller network of major roads (106.2 miles) and no rail infrastructure. It has 104 standalone lift stations and 56 bridges/culverts, along with 6 historic bridges, more than either of the other focus areas. Parks in the West account for 577.6 acres.

The “Outside” category, representing areas not included within the three focus areas, shows limited infrastructure and assets, with only 67.8 miles of major roads, 8.8 miles of rail, minimal counts of standalone lift stations (10), bridges/culverts (2), and no historic bridges. Parkland is also limited, at just 28.3 acres. These areas are not included in the focus area because they are not at risk of flooding for the scenarios noted as the basis for this work.

Overall, the distribution of infrastructure and community assets across the Adaptation Focus Areas highlights each region’s unique priorities: the Mid area serves as the main transportation and utility hub with dense development and social services; the South has more parks, rail connections, and resilient emergency facilities due to coastal risks; and the West balances community support with historic preservation. Areas outside the Adaptation Focus Areas have fewer assets overall, reflecting lower population and operational needs. These differences inform emergency response and development planning.

The following sections illustrate the details of each Adaptation Focus Area.

## Adaptation Focus Area 1 – South

### Focus Area Overview

This area includes southern Charlotte County shoreline communities that face high exposure to storm surge, tidal flooding, and sea level change. Coastal assets near Punta Gorda are highly sensitive to surge events, which may overwhelm neighborhoods, wastewater infrastructure, and road networks. River discharge from the Peace River can intensify flooding even in the absence of local storms, making this area vulnerable to compound events, even though the riverine flooding is not considered in this analysis. Critical facilities, transportation routes, and public safety buildings in this zone are essential for emergency response during major weather events.

The area contains a variety of critical facilities that support both emergency management and everyday community needs. Among these are fire stations, local government buildings, and health care centres, all strategically located to deliver efficient emergency response and essential services. The fire stations and public safety buildings are positioned in key locations to maximize coverage, ensuring swift action during major weather events, such as storm surges and flooding. Given the area's heightened vulnerability to coastal hazards, these facilities play a vital role in disaster preparedness and recovery. Their presence underscores the importance of resilient infrastructure in protecting residents and maintaining essential operations during emergencies.

### Impacted Facilities

The following table identifies the facility buildings within Adaptation Focus Area 1 – South that are projected to be affected and sustain damage due to 100-year storm surge events. These facilities, which include schools, fire stations, local government buildings, health care centers, and represent critical community and emergency assets. The preliminary ranking is decided by the estimated building losses (as described in Appendix D Technical Approach and Appendix E Exposure and Sensitivity Analysis). An asterisk next to an asset name indicates a 0.5 ft first floor elevation was assumed for estimating flood losses due to limited data. Loss estimates can be revised if more precise elevation information becomes available.

Table 5 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 1- South under Current Condition

| Rank | Asset Name | Asset Type | Asset Class | Longitude | Latitude | Area (sqft) | Potential Flood Depth (ft) | Estimated Damage Severity | Estimated 100-year Storm Event Losses ($) | Estimated 100-year Storm Event Losses Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0379 | 26.9274 | 263,316 | 1.1 | Minor | $30,229K | High |
| 2 | Baker Center Head Start | Schools | Critical Community And Emergency Facilities | -82.0433 | 26.9337 | 60,387 | 1.5 | Minor | $24,535K | High |
| 3 | Punta Gorda Middle School* | Schools | Critical Community And Emergency Facilities | -82.0342 | 26.9285 | 112,601 | 1.4 | Minor | $14,078K | Medium |
| 4 | Life Care Center Of Punta Gorda | Health Care Facilities | Critical Community And Emergency Facilities | -82.0557 | 26.9268 | 47,559 | 0.8 | Affected | $13,580K | Medium |
| 5 | Justice Center | Local Government Facilities | Critical Community And Emergency Facilities | -82.0474 | 26.9383 | 64,606 | 0.2 | Affected | $11,470K | Medium |
| 6 | Verandas Of Punta Gorda* | Affordable Public Housing | Critical Community And Emergency Facilities | -82.0403 | 26.9178 | 62,922 | 4.9 | Major | $9,051K | Low |
| 7 | Charlotte Harbor Event & Conf. Center | Local Government Facilities | Critical Community And Emergency Facilities | -82.0536 | 26.9370 | 50,151 | 4.3 | Major | $7,747K | Low |
| 8 | Punta Gorda Post Office | Communications Facilities | Critical Infrastructure | -82.0495 | 26.9372 | 27,378 | 3.7 | Major | $6,215K | Low |
| 9 | Charlotte Behavioral Health Care* | Health Care Facilities | Critical Community And Emergency Facilities | -82.0307 | 26.9233 | 22,522 | 1.6 | Minor | $5,963K | Low |
| 10 | Charlotte Regional Medical Center | Hospitals | Critical Community And Emergency Facilities | -82.0400 | 26.9409 | 103,909 | 0.6 | Affected | $4,842K | Minimal |
| 11 | Historic District | Historical And Cultural Assets | Natural, Cultural, And Historical Resources | -82.0403 | 26.9255 | 11,095 | 0.8 | Affected | $2,990K | Minimal |
| 12 | Punta Gorda Library | Community Centers | Critical Community And Emergency Facilities | -82.0532 | 26.9245 | 9,178 | 4.0 | Major | $2,596K | Minimal |
| 13 | Harborside Surgery Center | Health Care Facilities | Critical Community And Emergency Facilities | -82.0424 | 26.9392 | 6,734 | 3.4 | Major | $2,386K | Minimal |
| 14 | Sallie Jones Elementary School | Schools | Critical Community And Emergency Facilities | -82.0415 | 26.9280 | 88,868 | (0.4) |  | $2,319K | Minimal |
| 15 | Rai Care Centers - Punta Gorda | Health Care Facilities | Critical Community And Emergency Facilities | -82.0467 | 26.9354 | 6,593 | 2.3 | Major | $2,070K | Minimal |
| 16 | Police Station 1 | Law Enforcement Facilities, Fire Stations | Critical Community And Emergency Facilities | -82.0466 | 26.9255 | 21,246 | 0.6 | Affected | $1,699K | Minimal |
| 17 | Charlotte Community Mental Health Services | Health Care Facilities | Critical Community And Emergency Facilities | -82.0327 | 26.9237 | 16,273 | 0.3 | Affected | $1,496K | Minimal |
| 18 | Punta Gorda Fd St 3/17* | Fire Stations | Critical Community And Emergency Facilities | -82.0692 | 26.9094 | 3,793 | 3.2 | Major | $1,251K | Minimal |
| 19 | South County Annex | Local Government Facilities | Critical Community And Emergency Facilities | -82.0483 | 26.9332 | 30,413 | (0.9) |  | $1,128K | Minimal |
| 20 | Historic District* | Historical And Cultural Assets | Natural, Cultural, And Historical Resources | -82.0406 | 26.9254 | 2,332 | 0.7 | Affected | $558K | Minimal |
| 21 | Punta Gorda Fd St 2* | Fire Stations | Critical Community And Emergency Facilities | -82.0220 | 26.8840 | 7,402 | 0.6 | Affected | $541K | Minimal |
| 22 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0365 | 26.9276 | 4,186 | 1.1 | Minor | $488K | Minimal |
| 23 | South County Regional Park | Parks | Natural, Cultural, And Historical Resources | -82.0363 | 26.9333 | 3,438 | 0.4 | Affected | $459K | Minimal |
| 24 | Verandas Of Punta Gorda | Affordable Public Housing | Critical Community And Emergency Facilities | -82.0400 | 26.9175 | 1,248 | 0.7 | Affected | $382K | Minimal |
| 25 | Historic District | Historical And Cultural Assets | Natural, Cultural, And Historical Resources | -82.0402 | 26.9253 | 536 | 1.0 | Minor | $172K | Minimal |
| 26 | Sallie Jones Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0408 | 26.9280 | 5,132 | - |  | $137K | Minimal |
| 27 | Police Station 1 | Law Enforcement Facilities | Critical Community And Emergency Facilities | -82.0459 | 26.9253 | 2,624 | (0.2) |  | $101K | Minimal |
| 28 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0390 | 26.9275 | 2,179 | (0.4) |  | $57K | Minimal |
| 29 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0390 | 26.9273 | 1,247 | (0.3) |  | $33K | Minimal |
| 30 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0390 | 26.9276 | 1,171 | (0.3) |  | $31K | Minimal |

Table 6 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 1- South when Change in Sea Level Exceeds 1 ft

| Rank | Asset Name | Asset Type | Asset Class | Longitude | Latitude | Area (sqft) | Potential Flood Depth (ft) | Estimated Damage Severity | Estimated 100-year Storm Event Losses ($) | Estimated 100-year Storm Event Losses Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0379 | 26.9274 | 263,316 | 2.1 | Major | $40,262K | High |
| 2 | Justice Center | Local Government Facilities | Critical Community And Emergency Facilities | -82.0474 | 26.9383 | 64,606 | 1.3 | Minor | $38,851K | High |
| 3 | Baker Center Head Start | Schools | Critical Community And Emergency Facilities | -82.0433 | 26.9337 | 60,387 | 2.5 | Major | $25,866K | High |
| 4 | Life Care Center Of Punta Gorda | Health Care Facilities | Critical Community And Emergency Facilities | -82.0557 | 26.9268 | 47,559 | 1.8 | Minor | $24,755K | High |
| 5 | Sallie Jones Elementary School | Schools | Critical Community And Emergency Facilities | -82.0415 | 26.9280 | 88,868 | 0.7 | Affected | $24,698K | High |
| 6 | Punta Gorda Middle School* | Schools | Critical Community And Emergency Facilities | -82.0342 | 26.9285 | 112,601 | 2.4 | Major | $18,706K | High |
| 7 | Verandas Of Punta Gorda* | Affordable Public Housing | Critical Community And Emergency Facilities | -82.0403 | 26.9178 | 62,922 | 6.0 | Destroyed | $9,794K | Low |
| 8 | Charlotte Harbor Event & Conf. Center | Local Government Facilities | Critical Community And Emergency Facilities | -82.0536 | 26.9370 | 50,151 | 5.4 | Major | $8,462K | Low |
| 9 | Charlotte Behavioral Health Care* | Health Care Facilities | Critical Community And Emergency Facilities | -82.0307 | 26.9233 | 22,522 | 2.7 | Major | $7,498K | Low |
| 10 | Punta Gorda Post Office | Communications Facilities | Critical Infrastructure | -82.0495 | 26.9372 | 27,378 | 4.7 | Major | $6,819K | Low |
| 11 | Charlotte Regional Medical Center | Hospitals | Critical Community And Emergency Facilities | -82.0400 | 26.9409 | 103,909 | 1.6 | Minor | $5,223K | Low |
| 12 | Charlotte Community Mental Health Services | Health Care Facilities | Critical Community And Emergency Facilities | -82.0327 | 26.9237 | 16,273 | 1.3 | Minor | $3,785K | Minimal |
| 13 | Historic District | Historical And Cultural Assets | Natural, Cultural, And Historical Resources | -82.0403 | 26.9255 | 11,095 | 1.9 | Minor | $3,618K | Minimal |
| 14 | Police Station 1 | Law Enforcement Facilities, Fire Stations | Critical Community And Emergency Facilities | -82.0466 | 26.9255 | 21,246 | 1.6 | Minor | $3,044K | Minimal |
| 15 | Punta Gorda Library | Community Centers | Critical Community And Emergency Facilities | -82.0532 | 26.9245 | 9,178 | 5.0 | Major | $2,767K | Minimal |
| 16 | Harborside Surgery Center | Health Care Facilities | Critical Community And Emergency Facilities | -82.0424 | 26.9392 | 6,734 | 4.4 | Major | $2,534K | Minimal |
| 17 | Rai Care Centers - Punta Gorda | Health Care Facilities | Critical Community And Emergency Facilities | -82.0467 | 26.9354 | 6,593 | 3.3 | Major | $2,326K | Minimal |
| 18 | South County Annex | Local Government Facilities | Critical Community And Emergency Facilities | -82.0483 | 26.9332 | 30,413 | 0.1 | Affected | $2,234K | Minimal |
| 19 | Sallie Jones Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0408 | 26.9280 | 5,132 | 1.0 | Minor | $2,069K | Minimal |
| 20 | Punta Gorda Fd St 3/17* | Fire Stations | Critical Community And Emergency Facilities | -82.0692 | 26.9094 | 3,793 | 4.2 | Major | $1,708K | Minimal |
| 21 | Punta Gorda Fd St 2* | Fire Stations | Critical Community And Emergency Facilities | -82.0220 | 26.8840 | 7,402 | 1.6 | Minor | $1,125K | Minimal |
| 22 | South County Regional Park | Parks | Natural, Cultural, And Historical Resources | -82.0363 | 26.9333 | 3,438 | 1.4 | Minor | $769K | Minimal |
| 23 | Historic District* | Historical And Cultural Assets | Natural, Cultural, And Historical Resources | -82.0406 | 26.9254 | 2,332 | 1.7 | Minor | $759K | Minimal |
| 24 | Verandas Of Punta Gorda | Affordable Public Housing | Critical Community And Emergency Facilities | -82.0400 | 26.9175 | 1,248 | 1.7 | Minor | $731K | Minimal |
| 25 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0365 | 26.9276 | 4,186 | 2.2 | Major | $649K | Minimal |
| 26 | Florida Southwestern State College - Charlotte Campus | Colleges And Universities | Critical Community And Emergency Facilities | -82.0183 | 26.9237 | 6,223 | 0.2 | Affected | $632K | Minimal |
| 27 | Police Station 1 | Law Enforcement Facilities | Critical Community And Emergency Facilities | -82.0459 | 26.9253 | 2,624 | 0.8 | Affected | $501K | Minimal |
| 28 | Public Works | Local Government Facilities | Critical Community And Emergency Facilities | -82.0182 | 26.9293 | 9,954 | (0.4) |  | $381K | Minimal |
| 29 | Lighting District Administration* | Local Government Facilities | Critical Community And Emergency Facilities | -82.0177 | 26.9290 | 3,422 | 0.3 | Affected | $378K | Minimal |
| 30 | Florida Southwestren Collegiate High School - Charlotte Campus | Schools | Critical Community And Emergency Facilities | -82.0185 | 26.9245 | 13,187 | (0.4) |  | $344K | Minimal |
| 31 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0390 | 26.9275 | 2,179 | 0.7 | Affected | $179K | Minimal |
| 32 | Historic District | Historical And Cultural Assets | Natural, Cultural, And Historical Resources | -82.0402 | 26.9253 | 536 | 2.1 | Minor | $176K | Minimal |
| 33 | Public Works* | Local Government Facilities | Critical Community And Emergency Facilities | -82.0180 | 26.9295 | 1,917 | 0.2 | Affected | $176K | Minimal |
| 34 | Public Works* | Local Government Facilities | Critical Community And Emergency Facilities | -82.0174 | 26.9291 | 2,401 | 0.1 | Affected | $171K | Minimal |
| 35 | Public Works* | Local Government Facilities | Critical Community And Emergency Facilities | -82.0184 | 26.9298 | 919 | 0.4 | Affected | $136K | Minimal |
| 36 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0390 | 26.9273 | 1,247 | 0.8 | Affected | $114K | Minimal |
| 37 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0390 | 26.9276 | 1,171 | 0.8 | Affected | $109K | Minimal |

Table 7 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 1- South when Change in Sea Level Exceeds 3 ft

| Rank | Asset Name | Asset Type | Asset Class | Longitude | Latitude | Area (sqft) | Potential Flood Depth (ft) | Estimated Damage Severity | Estimated 100-year Storm Event Losses ($) | Estimated 100-year Storm Event Losses Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0379 | 26.9274 | 263,316 | 4.1 | Major | $59,200K | High |
| 2 | Justice Center | Local Government Facilities | Critical Community And Emergency Facilities | -82.0474 | 26.9383 | 64,606 | 3.3 | Major | $42,809K | High |
| 3 | Sallie Jones Elementary School | Schools | Critical Community And Emergency Facilities | -82.0415 | 26.9280 | 88,868 | 2.7 | Major | $38,540K | High |
| 4 | Life Care Center Of Punta Gorda | Health Care Facilities | Critical Community And Emergency Facilities | -82.0557 | 26.9268 | 47,559 | 3.8 | Major | $32,072K | High |
| 5 | Baker Center Head Start | Schools | Critical Community And Emergency Facilities | -82.0433 | 26.9337 | 60,387 | 4.5 | Major | $27,058K | High |
| 6 | Punta Gorda Middle School* | Schools | Critical Community And Emergency Facilities | -82.0342 | 26.9285 | 112,601 | 4.4 | Major | $25,775K | High |
| 7 | Charlotte Regional Medical Center | Hospitals | Critical Community And Emergency Facilities | -82.0400 | 26.9409 | 103,909 | 3.6 | Major | $24,873K | High |
| 8 | Verandas Of Punta Gorda* | Affordable Public Housing | Critical Community And Emergency Facilities | -82.0403 | 26.9178 | 62,922 | 7.9 | Destroyed | $11,210K | Medium |
| 9 | Charlotte Harbor Event & Conf. Center | Local Government Facilities | Critical Community And Emergency Facilities | -82.0536 | 26.9370 | 50,151 | 7.3 | Destroyed | $10,227K | Medium |
| 10 | Florida Southwestern State College - Charlotte Campus | Colleges And Universities | Critical Community And Emergency Facilities | -82.0187 | 26.9233 | 23,702 | 1.5 | Minor | $10,166K | Medium |
| 11 | South County Annex | Local Government Facilities | Critical Community And Emergency Facilities | -82.0483 | 26.9332 | 30,413 | 2.1 | Major | $10,049K | Medium |
| 12 | Florida Southwestern State College - Charlotte Campus | Colleges And Universities | Critical Community And Emergency Facilities | -82.0190 | 26.9228 | 21,229 | 1.3 | Minor | $9,076K | Low |
| 13 | Charlotte Behavioral Health Care* | Health Care Facilities | Critical Community And Emergency Facilities | -82.0307 | 26.9233 | 22,522 | 4.7 | Major | $8,632K | Low |
| 14 | Punta Gorda Post Office | Communications Facilities | Critical Infrastructure | -82.0495 | 26.9372 | 27,378 | 6.7 | Destroyed | $8,269K | Low |
| 15 | Florida Southwestern State College - Charlotte Campus | Colleges And Universities | Critical Community And Emergency Facilities | -82.0200 | 26.9220 | 17,803 | 1.5 | Minor | $7,634K | Low |
| 16 | Florida Southwestern State College - Charlotte Campus | Colleges And Universities | Critical Community And Emergency Facilities | -82.0192 | 26.9219 | 16,767 | 1.5 | Minor | $7,192K | Low |
| 17 | Florida Southwestern State College - Charlotte Campus | Colleges And Universities | Critical Community And Emergency Facilities | -82.0195 | 26.9223 | 14,626 | 1.3 | Minor | $6,256K | Low |
| 18 | Charlotte Community Mental Health Services | Health Care Facilities | Critical Community And Emergency Facilities | -82.0327 | 26.9237 | 16,273 | 3.3 | Major | $5,758K | Low |
| 19 | Police Station 1 | Law Enforcement Facilities, Fire Stations | Critical Community And Emergency Facilities | -82.0466 | 26.9255 | 21,246 | 3.6 | Major | $5,586K | Low |
| 20 | South County Regional Park | Parks | Natural, Cultural, And Historical Resources | -82.0368 | 26.9330 | 19,286 | 1.4 | Minor | $4,328K | Minimal |
| 21 | East Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0041 | 26.9407 | 89,820 | 0.1 | Affected | $4,309K | Minimal |
| 22 | Historic District | Historical And Cultural Assets | Natural, Cultural, And Historical Resources | -82.0403 | 26.9255 | 11,095 | 3.9 | Major | $3,966K | Minimal |
| 23 | Transit Department | Local Government Facilities | Critical Community And Emergency Facilities | -82.0244 | 26.9192 | 15,303 | 0.7 | Affected | $3,773K | Minimal |
| 24 | Public Works | Local Government Facilities | Critical Community And Emergency Facilities | -82.0182 | 26.9293 | 9,954 | 1.6 | Minor | $3,227K | Minimal |
| 25 | Punta Gorda Fd St 2* | Fire Stations | Critical Community And Emergency Facilities | -82.0220 | 26.8840 | 7,402 | 3.6 | Major | $2,877K | Minimal |
| 26 | Punta Gorda Library | Community Centers | Critical Community And Emergency Facilities | -82.0532 | 26.9245 | 9,178 | 7.0 | Destroyed | $2,860K | Minimal |
| 27 | Harborside Surgery Center | Health Care Facilities | Critical Community And Emergency Facilities | -82.0424 | 26.9392 | 6,734 | 6.4 | Destroyed | $2,808K | Minimal |
| 28 | Florida Southwestern State College - Charlotte Campus | Colleges And Universities | Critical Community And Emergency Facilities | -82.0183 | 26.9237 | 6,223 | 2.2 | Major | $2,725K | Minimal |
| 29 | Rai Care Centers - Punta Gorda | Health Care Facilities | Critical Community And Emergency Facilities | -82.0467 | 26.9354 | 6,593 | 5.3 | Major | $2,622K | Minimal |
| 30 | Sallie Jones Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0408 | 26.9280 | 5,132 | 3.0 | Major | $2,287K | Minimal |
| 31 | Punta Gorda Fd St 3/17* | Fire Stations | Critical Community And Emergency Facilities | -82.0692 | 26.9094 | 3,793 | 6.2 | Destroyed | $2,151K | Minimal |
| 32 | Florida Southwestern State College - Charlotte Campus | Colleges And Universities | Critical Community And Emergency Facilities | -82.0198 | 26.9226 | 4,623 | 1.2 | Minor | $1,973K | Minimal |
| 33 | Florida Southwestren Collegiate High School - Charlotte Campus | Schools | Critical Community And Emergency Facilities | -82.0185 | 26.9245 | 13,187 | 1.6 | Minor | $1,754K | Minimal |
| 34 | Police Station 1 | Law Enforcement Facilities | Critical Community And Emergency Facilities | -82.0459 | 26.9253 | 2,624 | 2.8 | Major | $1,420K | Minimal |
| 35 | Florida Southwestern State College - Charlotte Campus | Colleges And Universities | Critical Community And Emergency Facilities | -82.0190 | 26.9225 | 3,164 | 1.2 | Minor | $1,351K | Minimal |
| 36 | Lighting District Administration* | Local Government Facilities | Critical Community And Emergency Facilities | -82.0177 | 26.9290 | 3,422 | 2.3 | Major | $1,144K | Minimal |
| 37 | Verandas Of Punta Gorda | Affordable Public Housing | Critical Community And Emergency Facilities | -82.0400 | 26.9175 | 1,248 | 3.7 | Major | $1,137K | Minimal |
| 38 | South County Regional Park | Parks | Natural, Cultural, And Historical Resources | -82.0363 | 26.9333 | 3,438 | 3.4 | Major | $999K | Minimal |
| 39 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0365 | 26.9276 | 4,186 | 4.2 | Major | $944K | Minimal |
| 40 | Historic District* | Historical And Cultural Assets | Natural, Cultural, And Historical Resources | -82.0406 | 26.9254 | 2,332 | 3.7 | Major | $833K | Minimal |
| 41 | Public Works* | Local Government Facilities | Critical Community And Emergency Facilities | -82.0174 | 26.9291 | 2,401 | 2.1 | Major | $793K | Minimal |
| 42 | Florida Southwestren Collegiate High School - Charlotte Campus | Schools | Critical Community And Emergency Facilities | -82.0186 | 26.9242 | 6,014 | 1.5 | Minor | $778K | Minimal |
| 43 | East Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0036 | 26.9414 | 4,292 | 0.4 | Affected | $733K | Minimal |
| 44 | Public Works* | Local Government Facilities | Critical Community And Emergency Facilities | -82.0180 | 26.9295 | 1,917 | 2.2 | Major | $637K | Minimal |
| 45 | East Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0049 | 26.9411 | 17,345 | (0.2) |  | $458K | Minimal |
| 46 | Sheriff - Jail | Correctional Facilities | Critical Community And Emergency Facilities | -82.0060 | 26.9165 | 6,330 | 1.4 | Minor | $453K | Minimal |
| 47 | Florida Southwestern State College - Charlotte Campus | Colleges And Universities | Critical Community And Emergency Facilities | -82.0192 | 26.9232 | 1,006 | 1.3 | Minor | $430K | Minimal |
| 48 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0390 | 26.9275 | 2,179 | 2.7 | Major | $385K | Minimal |
| 49 | Public Works* | Local Government Facilities | Critical Community And Emergency Facilities | -82.0184 | 26.9298 | 919 | 2.4 | Major | $311K | Minimal |
| 50 | Sheriff - Jail | Correctional Facilities | Critical Community And Emergency Facilities | -82.0076 | 26.9159 | 6,086 | 0.7 | Affected | $282K | Minimal |
| 51 | Historic Courthouse | Historical And Cultural Assets | Natural, Cultural, And Historical Resources | -82.0502 | 26.9346 | 7,941 | (2.2) |  | $271K | Minimal |
| 52 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0390 | 26.9273 | 1,247 | 2.8 | Major | $227K | Minimal |
| 53 | Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.0390 | 26.9276 | 1,171 | 2.8 | Major | $214K | Minimal |
| 54 | Public Works* | Local Government Facilities | Critical Community And Emergency Facilities | -82.0180 | 26.9293 | 599 | 1.3 | Minor | $193K | Minimal |
| 55 | Historic District | Historical And Cultural Assets | Natural, Cultural, And Historical Resources | -82.0402 | 26.9253 | 536 | 4.1 | Major | $192K | Minimal |
| 56 | East Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0044 | 26.9411 | 5,935 | (0.2) |  | $157K | Minimal |

## Adaptation Focus Area 2 – Mid

### Focus Area Overview

Central Charlotte County includes the area surrounding the Port Charlotte and connected neighborhoods. This region contains a high concentration of critical assets identified in the VA, including government buildings, public safety facilities, utilities, and major transportation corridors. Its proximity to the Peace and Myakka Rivers, combined with rainfall-driven flooding and storm surge funneling into Charlotte Harbor, creates significant exposure to multi-hazard flooding. However, only coastal hazards were considered in this analysis.

Area mid stands out as the core hub for Charlotte County’s critical infrastructure and community services, housing the largest number of facility buildings among the Adaptation Action Areas. With 132 out of 256 total community and emergency facility buildings, Area mid is particularly notable for its concentration of schools, 74 buildings in total, highlighting its role as the county’s educational center. This area also contains the majority of health care facilities (13 of 21), including nearly all hospitals (7 of 8), as well as significant numbers of local government offices and community centers. Additionally, Area mid is the location for all lift stations and a substantial portion of pump stations, underlining its importance for both utility infrastructure and public service provision. The broad array of assets in Area mid reflects a dense, multifaceted service area that supports high population density and functions as an administrative and emergency response nucleus for the region.

### Impacted Facilities

Table 8 provides a detailed listing of facility buildings located in Adaptation Focus Area 2 – Mid that are anticipated to experience impacts and potential damage resulting from 100-year storm surge events. The facilities included encompass essential community and emergency assets such as affordable public housing, community centers, fire stations, wastewater treatment facilities, etc. The preliminary ranking is decided by the estimated building (as described in Appendix D Technical Approach and Appendix E Exposure and Sensitivity Analysis). An asterisk next to an asset name indicates a 0.5 ft first floor elevation was assumed for estimating flood losses due to limited data. Loss estimates can be revised if more precise elevation information becomes available.

Table 8 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 2 - Mid under Current Condition

| Rank | Asset Name | Asset Type | Asset Class | Longitude | Latitude | Area (sqft) | Potential Flood Depth (ft) | Estimated Damage Severity | Estimated 100-year Storm Event Losses ($) | Estimated 100-year Storm Event Losses Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Neil Armstrong Elementary School | Schools, Risk Shelter Inventory | Critical Community And Emergency Facilities | -82.0870 | 27.0035 | 91,561 | 0.1 | Affected | $10,956K | Medium |
| 2 | Meadow Park Elementary School | Schools | Critical Community And Emergency Facilities | -82.1247 | 26.9859 | 92,185 | 0.2 | Affected | $10,906K | Medium |
| 3 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0749 | 26.9693 | 24,279 | 0.6 | Affected | $6,085K | Low |
| 4 | Port Charlotte Artificial Kidney Center | Health Care Facilities | Critical Community And Emergency Facilities | -82.0685 | 26.9631 | 142,902 | (1.7) |  | $5,128K | Low |
| 5 | Marian Manor | Affordable Public Housing | Critical Community And Emergency Facilities | -82.0803 | 26.9658 | 20,510 | 2.2 | Major | $1,993K | Minimal |
| 6 | CCU Eastport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.0283 | 26.9778 | 36,206 | (1.2) |  | $1,319K | Minimal |
| 7 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0745 | 26.9693 | 4,777 | 0.4 | Affected | $791K | Minimal |
| 8 | Gulf Coast Marine Center Inc | Marinas | Transportation Assets And Evacuation Routes | -82.2097 | 26.9679 | 16,165 | (0.5) |  | $615K | Minimal |
| 9 | Charlotte County Fire & Ems St 3* | Fire Stations | Critical Community And Emergency Facilities | -82.2102 | 26.9662 | 4,377 | 1.5 | Minor | $588K | Minimal |
| 10 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities | -82.1123 | 26.9632 | 1,815 | 6.4 | Destroyed | $493K | Minimal |
| 11 | CCU Eastport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.0389 | 26.9855 | 9,808 | 0.3 | Affected | $408K | Minimal |
| 12 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0747 | 26.9696 | 929 | 1.0 | Minor | $374K | Minimal |
| 13 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0749 | 26.9696 | 853 | 1.2 | Minor | $345K | Minimal |
| 14 | Port Charlotte Post Office | Communications Facilities | Critical Infrastructure | -82.0883 | 26.9755 | 5,593 | 0.3 | Affected | $340K | Minimal |
| 15 | P.C. Beach Complex | Community Centers | Critical Community And Emergency Facilities | -82.1128 | 26.9621 | 8,944 | (1.6) |  | $324K | Minimal |
| 16 | St. Charles Housing Ii | Affordable Public Housing | Critical Community And Emergency Facilities | -82.0816 | 26.9661 | 14,493 | 0.1 | Affected | $264K | Minimal |
| 17 | Charlotte County Fire & Ems St 1 | Fire Stations | Critical Community And Emergency Facilities | -82.0921 | 26.9762 | 8,559 | (3.9) |  | $256K | Minimal |
| 18 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities | -82.1122 | 26.9619 | 876 | 5.8 | Destroyed | $225K | Minimal |
| 19 | Charlotte Harbor School* | Schools | Critical Community And Emergency Facilities | -82.0770 | 26.9686 | 4,722 | - | Affected | $192K | Minimal |
| 20 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0738 | 26.9695 | 5,439 | (0.5) |  | $141K | Minimal |
| 21 | Meadow Park Elementary School | Schools | Critical Community And Emergency Facilities | -82.1243 | 26.9864 | 4,959 | (0.4) |  | $129K | Minimal |
| 22 | Meadow Park Elementary School | Schools | Critical Community And Emergency Facilities | -82.1237 | 26.9868 | 4,732 | (0.5) |  | $123K | Minimal |
| 23 | Meadow Park Elementary School | Schools | Critical Community And Emergency Facilities | -82.1247 | 26.9863 | 4,335 | (0.6) |  | $111K | Minimal |
| 24 | Gleneagles Water Booster Station* | Pump Stations | Critical Infrastructure | -82.0797 | 26.9892 | 2,417 | - |  | $90K | Minimal |
| 25 | P.C. Beach Complex | Community Centers | Critical Community And Emergency Facilities | -82.1121 | 26.9617 | 1,663 | 0.1 | Affected | $84K | Minimal |
| 26 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities | -82.1108 | 26.9619 | 578 | 3.4 | Major | $79K | Minimal |
| 27 | Ambitrans Medical Transport | Emergency Medical Service Facilities | Critical Community And Emergency Facilities | -82.0871 | 26.9759 | 2,287 | (3.1) |  | $77K | Minimal |
| 28 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities | -82.1112 | 26.9617 | 196 | 5.0 | Major | $43K | Minimal |
| 29 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0738 | 26.9689 | 865 | 0.1 | Affected | $41K | Minimal |
| 30 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities | -82.1117 | 26.9617 | 137 | 5.4 | Major | $32K | Minimal |
| 31 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0739 | 26.9690 | 899 | (0.1) |  | $24K | Minimal |
| 32 | Gleneagles Water Booster Station* | Pump Stations | Critical Infrastructure | -82.0795 | 26.9892 | 405 | 0.6 | Affected | $16K | Minimal |
| 33 | Gleneagles Water Booster Station* | Pump Stations | Critical Infrastructure | -82.0796 | 26.9890 | 153 | (0.2) |  | $5K | Minimal |

Table 9 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 2 - Mid when Change in Sea Level Exceeds 1 ft

| Rank | Asset Name | Asset Type |  | Asset Class |  | Longitude |  | Latitude |  | Area (sqft) | Potential Flood Depth (ft) |  |  | Estimated Damage Severity |  | Estimated 100-year Storm Event Losses ($) |  | Estimated 100-year Storm Event Losses Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Neil Armstrong Elementary School | Schools, Risk Shelter Inventory | Critical Community And Emergency Facilities |  | -82.0870 |  | 27.0035 |  | 91,561 |  |  | 1.2 | Minor |  | $71,328K |  | High |  |
| 2 | Deep Creek Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0107 |  | 26.9856 |  | 114,838 |  |  | 0.9 | Minor |  | $40,907K |  | High |  |
| 3 | Meadow Park Elementary School | Schools | Critical Community And Emergency Facilities |  | -82.1247 |  | 26.9859 |  | 92,185 |  |  | 1.3 | Minor |  | $37,319K |  | High |  |
| 4 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0749 |  | 26.9693 |  | 24,279 |  |  | 1.6 | Minor |  | $9,890K |  | Low |  |
| 5 | Port Charlotte Artificial Kidney Center | Health Care Facilities | Critical Community And Emergency Facilities |  | -82.0685 |  | 26.9631 |  | 142,902 |  |  | (0.7) |  |  | $5,753K |  | Low |  |
| 6 | Fawcett Memorial Hospital | Hospitals | Critical Community And Emergency Facilities |  | -82.0982 |  | 26.9886 |  | 125,200 |  |  | - |  |  | $5,570K |  | Low |  |
| 7 | Family Services | Local Government Facilities | Critical Community And Emergency Facilities |  | -82.0919 |  | 26.9956 |  | 20,306 |  |  | 0.4 | Affected |  | $2,859K |  | Minimal |  |
| 8 | Major League Clubhouse | Parks | Natural, Cultural, And Historical Resources |  | -82.1806 |  | 26.9989 |  | 34,383 |  |  | (0.6) |  |  | $2,765K |  | Minimal |  |
| 9 | Marian Manor | Affordable Public Housing | Critical Community And Emergency Facilities |  | -82.0803 |  | 26.9658 |  | 20,510 |  |  | 3.2 | Major |  | $2,419K |  | Minimal |  |
| 10 | Community Christian School | Schools | Critical Community And Emergency Facilities |  | -82.1222 |  | 27.0100 |  | 20,758 |  |  | 1.1 | Minor |  | $2,366K |  | Minimal |  |
| 11 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0745 |  | 26.9693 |  | 4,777 |  |  | 1.4 | Minor |  | $1,938K |  | Minimal |  |
| 12 | Charlotte Harbor School* | Schools | Critical Community And Emergency Facilities |  | -82.0770 |  | 26.9686 |  | 4,722 |  |  | 1.1 | Minor |  | $1,905K |  | Minimal |  |
| 13 | Community Christian School* | Schools | Critical Community And Emergency Facilities |  | -82.1227 |  | 27.0100 |  | 20,712 |  |  | 0.7 | Affected |  | $1,849K |  | Minimal |  |
| 14 | CCU Eastport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure |  | -82.0283 |  | 26.9778 |  | 36,206 |  |  | (0.2) |  |  | $1,407K |  | Minimal |  |
| 15 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0738 |  | 26.9695 |  | 5,439 |  |  | 0.5 | Affected |  | $1,261K |  | Minimal |  |
| 16 | Meadow Park Elementary School | Schools | Critical Community And Emergency Facilities |  | -82.1243 |  | 26.9864 |  | 4,959 |  |  | 0.6 | Affected |  | $1,261K |  | Minimal |  |
| 17 | Gulf Coast Marine Center Inc | Marinas | Transporation Assets And Evacuation Routes |  | -82.2097 |  | 26.9679 |  | 16,165 |  |  | 0.5 | Affected |  | $1,179K |  | Minimal |  |
| 18 | Family Services* | Local Government Facilities | Critical Community And Emergency Facilities |  | -82.0926 |  | 26.9956 |  | 22,526 |  |  | - | Affected |  | $1,144K |  | Minimal |  |
| 19 | CCU Eastport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure |  | -82.0270 |  | 26.9797 |  | 29,043 |  |  | (0.2) |  |  | $1,128K |  | Minimal |  |
| 20 | Meadow Park Elementary School | Schools | Critical Community And Emergency Facilities |  | -82.1237 |  | 26.9868 |  | 4,732 |  |  | 0.5 | Affected |  | $1,084K |  | Minimal |  |
| 21 | Charlotte County Fire & Ems St 3* | Fire Stations | Critical Community And Emergency Facilities |  | -82.2102 |  | 26.9662 |  | 4,377 |  |  | 2.5 | Major |  | $1,062K |  | Minimal |  |
| 22 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0743 |  | 26.9690 |  | 7,363 |  |  | 0.3 | Affected |  | $1,060K |  | Minimal |  |
| 23 | St. Charles Housing Ii | Affordable Public Housing | Critical Community And Emergency Facilities |  | -82.0816 |  | 26.9661 |  | 14,493 |  |  | 1.1 | Minor |  | $1,011K |  | Minimal |  |
| 24 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0737 |  | 26.9692 |  | 4,718 |  |  | 0.4 | Affected |  | $840K |  | Minimal |  |
| 25 | Meadow Park Elementary School | Schools | Critical Community And Emergency Facilities |  | -82.1247 |  | 26.9863 |  | 4,335 |  |  | 0.4 | Affected |  | $788K |  | Minimal |  |
| 26 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0747 |  | 26.9689 |  | 4,870 |  |  | 0.4 | Affected |  | $775K |  | Minimal |  |
| 27 | Port Charlotte Post Office | Communications Facilities | Critical Infrastructure |  | -82.0883 |  | 26.9755 |  | 5,593 |  |  | 1.4 | Minor |  | $692K |  | Minimal |  |
| 28 | Building B | Parks | Natural, Cultural, And Historical Resources |  | -82.1800 |  | 26.9983 |  | 14,945 |  |  | (1.0) |  |  | $615K |  | Minimal |  |
| 29 | Fawcett Memorial Hospital | Hospitals | Critical Community And Emergency Facilities |  | -82.0993 |  | 26.9891 |  | 12,859 |  |  | (0.4) |  |  | $554K |  | Minimal |  |
| 30 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0736 |  | 26.9694 |  | 3,873 |  |  | 0.3 | Affected |  | $519K |  | Minimal |  |
| 31 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities |  | -82.1123 |  | 26.9632 |  | 1,815 |  |  | 7.4 | Destroyed |  | $514K |  | Minimal |  |
| 32 | CCU Eastport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure |  | -82.0389 |  | 26.9855 |  | 9,808 |  |  | 1.4 | Minor |  | $484K |  | Minimal |  |
| 33 | Gulf Coast Dialysis | Health Care Facilities | Critical Community And Emergency Facilities |  | -82.0991 |  | 26.9829 |  | 11,653 |  |  | (1.1) |  |  | $436K |  | Minimal |  |
| 34 | P.C. Beach Complex | Community Centers | Critical Community And Emergency Facilities |  | -82.1128 |  | 26.9621 |  | 8,944 |  |  | (0.6) |  |  | $400K |  | Minimal |  |
| 35 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0747 |  | 26.9696 |  | 929 |  |  | 2.1 | Minor |  | $383K |  | Minimal |  |
| 36 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0749 |  | 26.9696 |  | 853 |  |  | 2.3 | Major |  | $358K |  | Minimal |  |
| 37 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0738 |  | 26.9689 |  | 865 |  |  | 1.1 | Minor |  | $349K |  | Minimal |  |
| 38 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0739 |  | 26.9690 |  | 899 |  |  | 0.9 | Minor |  | $336K |  | Minimal |  |
| 39 | Community Christian School* | Schools | Critical Community And Emergency Facilities |  | -82.1225 |  | 27.0102 |  | 2,494 |  |  | 1.0 | Minor |  | $278K |  | Minimal |  |
| 40 | Charlotte County Fire & Ems St 1 | Fire Stations | Critical Community And Emergency Facilities |  | -82.0921 |  | 26.9762 |  | 8,559 |  |  | (2.9) |  |  | $277K |  | Minimal |  |
| 41 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities |  | -82.1122 |  | 26.9619 |  | 876 |  |  | 6.8 | Destroyed |  | $245K |  | Minimal |  |
| 42 | Community Christian School* | Schools | Critical Community And Emergency Facilities |  | -82.1226 |  | 27.0096 |  | 9,120 |  |  | (0.3) |  |  | $240K |  | Minimal |  |
| 43 | Charlotte County Fire & Ems St 15 | Fire Stations | Critical Community And Emergency Facilities |  | -82.2330 |  | 27.0103 |  | 6,378 |  |  | (0.8) |  |  | $239K |  | Minimal |  |
| 44 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0737 |  | 26.9690 |  | 934 |  |  | 0.4 | Affected |  | $181K |  | Minimal |  |
| 45 | Fawcett Memorial Hospital | Hospitals | Critical Community And Emergency Facilities |  | -82.0985 |  | 26.9895 |  | 2,653 |  |  | (0.4) |  |  | $115K |  | Minimal |  |
| 46 | P.C. Beach Complex | Community Centers | Critical Community And Emergency Facilities |  | -82.1121 |  | 26.9617 |  | 1,663 |  |  | 1.1 | Minor |  | $112K |  | Minimal |  |
| 47 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities |  | -82.1108 |  | 26.9619 |  | 578 |  |  | 4.5 | Major |  | $108K |  | Minimal |  |
| 48 | Gleneagles Water Booster Station* | Pump Stations | Critical Infrastructure |  | -82.0797 |  | 26.9892 |  | 2,417 |  |  | 1.0 | Minor |  | $103K |  | Minimal |  |
| 49 | Community Christian School* | Schools | Critical Community And Emergency Facilities |  | -82.1225 |  | 27.0103 |  | 838 |  |  | 1.0 | Minor |  | $92K |  | Minimal |  |
| 50 | Ambitrans Medical Transport | Emergency Medical Service Facilities | Critical Community And Emergency Facilities |  | -82.0871 |  | 26.9759 |  | 2,287 |  |  | (2.1) |  |  | $85K |  | Minimal |  |
| 51 | Community Christian School* | Schools | Critical Community And Emergency Facilities |  | -82.1226 |  | 27.0102 |  | 1,048 |  |  | 0.6 | Affected |  | $82K |  | Minimal |  |
| 52 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities |  | -82.0738 |  | 26.9691 |  | 947 |  |  | 0.1 | Affected |  | $73K |  | Minimal |  |
| 53 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities |  | -82.1112 |  | 26.9617 |  | 196 |  |  | 6.1 | Destroyed |  | $52K |  | Minimal |  |
| 54 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities |  | -82.1117 |  | 26.9617 |  | 137 |  |  | 6.4 | Destroyed |  | $37K |  | Minimal |  |
| 55 | Gleneagles Water Booster Station* | Pump Stations | Critical Infrastructure |  | -82.0795 |  | 26.9892 |  | 405 |  |  | 1.6 | Minor |  | $20K |  | Minimal |  |
| 56 | Gleneagles Water Booster Station* | Pump Stations | Critical Infrastructure |  | -82.0796 |  | 26.9890 |  | 153 |  |  | 0.8 | Affected |  | $6K |  | Minimal |  |

Table 10 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 2 - Mid when Change in Sea Level Exceeds 3 ft

| Rank | Asset Name | Asset Type | Asset Class | Longitude | Latitude | Area (sqft) | Potential Flood Depth (ft) | Estimated Damage Severity | Estimated 100-year Storm Event Losses ($) | Estimated 100-year Storm Event Losses Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Neil Armstrong Elementary School | Schools, Risk Shelter Inventory | Critical Community And Emergency Facilities | -82.0870 | 27.0035 | 91,561 | 3.2 | Major | $78,628K | High |
| 2 | Deep Creek Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0107 | 26.9856 | 114,838 | 2.9 | Major | $50,708K | High |
| 3 | Meadow Park Elementary School | Schools | Critical Community And Emergency Facilities | -82.1247 | 26.9859 | 92,185 | 3.3 | Major | $41,118K | High |
| 4 | Port Charlotte Artificial Kidney Center | Health Care Facilities | Critical Community And Emergency Facilities | -82.0685 | 26.9631 | 142,902 | 1.3 | Minor | $33,117K | High |
| 5 | Medical Examiner* | Health Care Facilities | Critical Community And Emergency Facilities | -82.1450 | 27.0148 | 130,784 | 1.2 | Minor | $28,991K | High |
| 6 | Community Development | Local Government Facilities | Critical Community And Emergency Facilities | -82.1420 | 27.0078 | 24,480 | 1.4 | Minor | $14,743K | Medium |
| 7 | County Administration* | Local Government Facilities | Critical Community And Emergency Facilities | -82.1411 | 27.0086 | 43,124 | 1.6 | Minor | $13,996K | Medium |
| 8 | Major League Clubhouse | Parks | Natural, Cultural, And Historical Resources | -82.1806 | 26.9989 | 34,383 | 1.4 | Minor | $13,632K | Medium |
| 9 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0749 | 26.9693 | 24,279 | 3.6 | Major | $10,846K | Medium |
| 10 | Port Charlotte Library | Disaster Recovery Centers, Community Centers | Critical Community And Emergency Facilities | -82.0991 | 26.9929 | 120,312 | 0.5 | Affected | $9,405K | Low |
| 11 | Peace River Regional Medical Ctr | Hospitals | Critical Community And Emergency Facilities | -82.0956 | 26.9891 | 189,011 | 0.5 | Affected | $8,748K | Low |
| 12 | Economic Development | Local Government Facilities | Critical Community And Emergency Facilities | -82.1422 | 27.0100 | 12,649 | 3.6 | Major | $8,393K | Low |
| 13 | Port Charlotte Middle School | Schools | Critical Community And Emergency Facilities | -82.0763 | 27.0004 | 134,244 | 0.3 | Affected | $7,545K | Low |
| 14 | Family Services* | Local Government Facilities | Critical Community And Emergency Facilities | -82.0926 | 26.9956 | 22,526 | 2.0 | Minor | $7,393K | Low |
| 15 | Family Services | Local Government Facilities | Critical Community And Emergency Facilities | -82.0919 | 26.9956 | 20,306 | 2.4 | Major | $6,845K | Low |
| 16 | Fawcett Memorial Hospital | Hospitals | Critical Community And Emergency Facilities | -82.0982 | 26.9886 | 125,200 | 2.0 | Minor | $6,455K | Low |
| 17 | Fleet Management | Local Government Facilities | Critical Community And Emergency Facilities | -82.1511 | 27.0172 | 18,758 | 2.3 | Major | $6,277K | Low |
| 18 | St Charles School* | Schools | Critical Community And Emergency Facilities | -82.0925 | 26.9895 | 54,329 | 0.8 | Affected | $4,947K | Minimal |
| 19 | Tidewell Hospice Inc | Health Care Facilities | Critical Community And Emergency Facilities | -82.1156 | 27.0131 | 18,863 | 1.3 | Minor | $4,390K | Minimal |
| 20 | Community Christian School | Schools | Critical Community And Emergency Facilities | -82.1222 | 27.0100 | 20,758 | 3.1 | Major | $4,045K | Minimal |
| 21 | Port Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.1501 | 27.0048 | 134,338 | - | Affected | $4,031K | Minimal |
| 22 | Stadium | Parks | Natural, Cultural, And Historical Resources | -82.1821 | 26.9988 | 41,981 | - |  | $3,781K | Minimal |
| 23 | Community Christian School* | Schools | Critical Community And Emergency Facilities | -82.1227 | 27.0100 | 20,712 | 2.7 | Major | $3,740K | Minimal |
| 24 | Murdock Post Office | Communications Facilities | Critical Infrastructure | -82.1357 | 27.0062 | 38,307 | 0.7 | Affected | $3,163K | Minimal |
| 25 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0743 | 26.9690 | 7,363 | 2.3 | Major | $3,100K | Minimal |
| 26 | Building B | Parks | Natural, Cultural, And Historical Resources | -82.1800 | 26.9983 | 14,945 | 1.0 | Minor | $3,073K | Minimal |
| 27 | Marian Manor | Affordable Public Housing | Critical Community And Emergency Facilities | -82.0803 | 26.9658 | 20,510 | 5.2 | Major | $3,028K | Minimal |
| 28 | Port Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.1497 | 27.0055 | 24,381 | 1.2 | Minor | $2,896K | Minimal |
| 29 | St Lucy'S Outpatient Surgical Center | Health Care Facilities | Critical Community And Emergency Facilities | -82.0985 | 26.9873 | 17,789 | 0.6 | Affected | $2,492K | Minimal |
| 30 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0738 | 26.9695 | 5,439 | 2.5 | Major | $2,335K | Minimal |
| 31 | Port Charlotte Annex Post Office | Communications Facilities | Critical Infrastructure | -82.1497 | 27.0173 | 20,561 | 0.6 | Affected | $2,273K | Minimal |
| 32 | Gulf Coast Dialysis | Health Care Facilities | Critical Community And Emergency Facilities | -82.0991 | 26.9829 | 11,653 | 0.9 | Minor | $2,183K | Minimal |
| 33 | Meadow Park Elementary School | Schools | Critical Community And Emergency Facilities | -82.1243 | 26.9864 | 4,959 | 2.6 | Major | $2,139K | Minimal |
| 34 | Gulf Coast Marine Center Inc | Marinas | Transportation Assets And Evacuation Routes | -82.2097 | 26.9679 | 16,165 | 2.5 | Major | $2,136K | Minimal |
| 35 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0745 | 26.9693 | 4,777 | 3.4 | Major | $2,132K | Minimal |
| 36 | Tandem Health Care Of Port Charlotte | Health Care Facilities | Critical Community And Emergency Facilities | -82.1411 | 27.0044 | 50,370 | (0.4) |  | $2,124K | Minimal |
| 37 | Charlotte Harbor School* | Schools | Critical Community And Emergency Facilities | -82.0770 | 26.9686 | 4,722 | 3.1 | Major | $2,104K | Minimal |
| 38 | Charlotte County Fire & Ems St 3* | Fire Stations | Critical Community And Emergency Facilities | -82.2102 | 26.9662 | 4,377 | 4.5 | Major | $2,061K | Minimal |
| 39 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0747 | 26.9689 | 4,870 | 2.4 | Major | $2,058K | Minimal |
| 40 | Meadow Park Elementary School | Schools | Critical Community And Emergency Facilities | -82.1237 | 26.9868 | 4,732 | 2.5 | Major | $2,030K | Minimal |
| 41 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0737 | 26.9692 | 4,718 | 2.4 | Major | $2,002K | Minimal |
| 42 | Charlotte Harbor School | Schools | Critical Community And Emergency Facilities | -82.0770 | 26.9690 | 91,676 | (3.1) |  | $1,994K | Minimal |
| 43 | CCU Eastport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.0283 | 26.9778 | 36,206 | 1.8 | Minor | $1,921K | Minimal |
| 44 | Meadow Park Elementary School | Schools | Critical Community And Emergency Facilities | -82.1247 | 26.9863 | 4,335 | 2.4 | Major | $1,841K | Minimal |
| 45 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1451 | 27.0063 | 26,667 | 0.5 | Affected | $1,800K | Minimal |
| 46 | Port Charlotte High School* | Schools | Critical Community And Emergency Facilities | -82.1509 | 27.0056 | 53,158 | 0.1 | Affected | $1,775K | Minimal |
| 47 | St. Charles Housing Ii | Affordable Public Housing | Critical Community And Emergency Facilities | -82.0816 | 26.9661 | 14,493 | 3.1 | Major | $1,675K | Minimal |
| 48 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0736 | 26.9694 | 3,873 | 2.3 | Major | $1,627K | Minimal |
| 49 | Medical Examiner | Health Care Facilities | Critical Community And Emergency Facilities | -82.1490 | 27.0172 | 7,961 | 1.0 | Minor | $1,581K | Minimal |
| 50 | CCU Eastport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.0270 | 26.9797 | 29,043 | 1.8 | Minor | $1,537K | Minimal |
| 51 | Community Christian School* | Schools | Critical Community And Emergency Facilities | -82.1226 | 27.0096 | 9,120 | 1.7 | Minor | $1,255K | Minimal |
| 52 | Building A | Parks | Natural, Cultural, And Historical Resources | -82.1811 | 26.9984 | 8,935 | 0.4 | Affected | $1,252K | Minimal |
| 53 | Port Charlotte Post Office | Communications Facilities | Critical Infrastructure | -82.0883 | 26.9755 | 5,593 | 3.4 | Major | $1,239K | Minimal |
| 54 | Fawcett Memorial Hospital | Hospitals | Critical Community And Emergency Facilities | -82.0994 | 26.9878 | 24,112 | 1.3 | Minor | $1,185K | Minimal |
| 55 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1472 | 27.0058 | 31,756 | 0.1 | Affected | $1,103K | Minimal |
| 56 | Mid County Regional Library | Community Centers | Critical Community And Emergency Facilities | -82.1224 | 27.0014 | 41,110 | (0.1) |  | $1,089K | Minimal |
| 57 | St. Charles Housing | Affordable Public Housing | Critical Community And Emergency Facilities | -82.0932 | 26.9885 | 13,976 | 1.3 | Minor | $1,034K | Minimal |
| 58 | Homestand & Kid'S Clubhouse Concession | Parks | Natural, Cultural, And Historical Resources | -82.1819 | 26.9983 | 4,293 | 1.6 | Minor | $995K | Minimal |
| 59 | Promenades Surgery Center | Health Care Facilities | Critical Community And Emergency Facilities | -82.1006 | 26.9840 | 15,359 | 0.1 | Affected | $942K | Minimal |
| 60 | Community Christian School* | Schools | Critical Community And Emergency Facilities | -82.1225 | 27.0093 | 10,490 | 0.7 | Affected | $903K | Minimal |
| 61 | CCU Eastport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.0319 | 26.9836 | 15,960 | 1.3 | Minor | $780K | Minimal |
| 62 | Ticket Sales & First Aid - Stadium | Parks | Natural, Cultural, And Historical Resources | -82.1824 | 26.9984 | 3,782 | 1.0 | Minor | $774K | Minimal |
| 63 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1472 | 27.0055 | 9,494 | 0.6 | Affected | $766K | Minimal |
| 64 | Port Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.1507 | 27.0049 | 27,574 | (0.4) |  | $719K | Minimal |
| 65 | Charlotte County Fire & Ems St 15 | Fire Stations | Critical Community And Emergency Facilities | -82.2330 | 27.0103 | 6,378 | 1.2 | Minor | $714K | Minimal |
| 66 | P.C. Beach Complex | Community Centers | Critical Community And Emergency Facilities | -82.1128 | 26.9621 | 8,944 | 1.4 | Minor | $653K | Minimal |
| 67 | CCU Eastport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.0389 | 26.9855 | 9,808 | 3.4 | Major | $649K | Minimal |
| 68 | Sheriff - District 3 | Law Enforcement Facilities | Critical Community And Emergency Facilities | -82.0988 | 26.9844 | 7,690 | 0.7 | Affected | $646K | Minimal |
| 69 | Fawcett Memorial Hospital | Hospitals | Critical Community And Emergency Facilities | -82.0993 | 26.9891 | 12,859 | 1.6 | Minor | $645K | Minimal |
| 70 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities | -82.1123 | 26.9632 | 1,815 | 9.4 | Destroyed | $543K | Minimal |
| 71 | Port Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.1491 | 27.0049 | 3,119 | 0.2 | Affected | $512K | Minimal |
| 72 | Port Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.1491 | 27.0052 | 1,515 | 0.4 | Affected | $496K | Minimal |
| 73 | Community Christian School* | Schools | Critical Community And Emergency Facilities | -82.1225 | 27.0102 | 2,494 | 3.0 | Major | $481K | Minimal |
| 74 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1464 | 27.0060 | 7,337 | 0.5 | Affected | $477K | Minimal |
| 75 | St Charles School | Schools | Critical Community And Emergency Facilities | -82.0934 | 26.9895 | 17,222 | (0.1) |  | $457K | Minimal |
| 76 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0747 | 26.9696 | 929 | 4.1 | Major | $416K | Minimal |
| 77 | Fawcett Memorial Hospital | Hospitals | Critical Community And Emergency Facilities | -82.0984 | 26.9893 | 8,080 | 1.4 | Minor | $399K | Minimal |
| 78 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0739 | 26.9690 | 899 | 2.9 | Major | $398K | Minimal |
| 79 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0737 | 26.9690 | 934 | 2.4 | Major | $398K | Minimal |
| 80 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0738 | 26.9691 | 947 | 2.1 | Major | $393K | Minimal |
| 81 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0738 | 26.9689 | 865 | 3.1 | Major | $386K | Minimal |
| 82 | Peace River Elementary School* | Schools | Critical Community And Emergency Facilities | -82.0749 | 26.9696 | 853 | 4.3 | Major | $382K | Minimal |
| 83 | St Charles School* | Schools | Critical Community And Emergency Facilities | -82.0916 | 26.9889 | 3,135 | 1.3 | Minor | $382K | Minimal |
| 84 | St Charles School* | Schools | Critical Community And Emergency Facilities | -82.0920 | 26.9898 | 5,731 | 0.5 | Affected | $377K | Minimal |
| 85 | St Charles School* | Schools | Critical Community And Emergency Facilities | -82.0922 | 26.9888 | 2,904 | 1.4 | Minor | $367K | Minimal |
| 86 | Charlotte County Fire & Ems St 1 | Fire Stations | Critical Community And Emergency Facilities | -82.0921 | 26.9762 | 8,559 | (0.9) |  | $317K | Minimal |
| 87 | Groundskeeping Maintenance Building C | Parks | Natural, Cultural, And Historical Resources | -82.1815 | 26.9978 | 3,009 | 0.1 | Affected | $312K | Minimal |
| 88 | Transit Authority* | Local Government Facilities | Critical Community And Emergency Facilities | -82.1284 | 27.0195 | 5,350 | 0.1 | Affected | $312K | Minimal |
| 89 | Presbyterian Homes Of Charlotte | Affordable Public Housing | Critical Community And Emergency Facilities | -82.1014 | 26.9906 | 16,119 | 0.1 | Affected | $282K | Minimal |
| 90 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1466 | 27.0051 | 10,567 | (0.3) |  | $277K | Minimal |
| 91 | Charlotte County Fire & Ems St 2* | Fire Stations | Critical Community And Emergency Facilities | -82.1568 | 27.0045 | 6,980 | (0.1) |  | $273K | Minimal |
| 92 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities | -82.1122 | 26.9619 | 876 | 8.8 | Destroyed | $260K | Minimal |
| 93 | CCU Eastport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.0345 | 26.9841 | 5,541 | 0.9 | Minor | $255K | Minimal |
| 94 | P.C. Beach Complex | Community Centers | Critical Community And Emergency Facilities | -82.1121 | 26.9617 | 1,663 | 3.1 | Major | $205K | Minimal |
| 95 | Gertrude Water Booster Station | Pump Stations | Critical Infrastructure | -82.1026 | 26.9918 | 4,173 | 1.3 | Minor | $191K | Minimal |
| 96 | Charlotte County Fire & Ems St 11 | Fire Stations | Critical Community And Emergency Facilities | -82.0066 | 26.9876 | 4,863 | (0.6) |  | $184K | Minimal |
| 97 | Community Christian School* | Schools | Critical Community And Emergency Facilities | -82.1226 | 27.0102 | 1,048 | 2.6 | Major | $183K | Minimal |
| 98 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1475 | 27.0051 | 6,628 | (0.4) |  | $173K | Minimal |
| 99 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1468 | 27.0055 | 6,399 | - |  | $171K | Minimal |
| 100 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1457 | 27.0064 | 1,342 | 1.3 | Minor | $165K | Minimal |
| 101 | Community Christian School* | Schools | Critical Community And Emergency Facilities | -82.1225 | 27.0103 | 838 | 3.0 | Major | $160K | Minimal |
| 102 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities | -82.1108 | 26.9619 | 578 | 6.5 | Destroyed | $158K | Minimal |
| 103 | Gleneagles Water Booster Station* | Pump Stations | Critical Infrastructure | -82.0797 | 26.9892 | 2,417 | 3.0 | Major | $146K | Minimal |
| 104 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1463 | 27.0050 | 5,244 | (0.2) |  | $138K | Minimal |
| 105 | CCU Eastport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.0361 | 26.9833 | 3,128 | 0.2 | Affected | $134K | Minimal |
| 106 | Fawcett Memorial Hospital | Hospitals | Critical Community And Emergency Facilities | -82.0985 | 26.9895 | 2,653 | 1.6 | Minor | $133K | Minimal |
| 107 | Port Charlotte High School | Schools | Critical Community And Emergency Facilities | -82.1510 | 27.0048 | 1,555 | 0.1 | Affected | $122K | Minimal |
| 108 | Fawcett Memorial Hospital | Hospitals | Critical Community And Emergency Facilities | -82.0983 | 26.9895 | 2,446 | 1.3 | Minor | $121K | Minimal |
| 109 | Charlotte County Lf #2/Wtcc-71676 | Solid And Hazardous Waste Facilities | Critical Infrastructure | -82.0443 | 26.9841 | 24,888 | - | Affected | $109K | Minimal |
| 110 | Ambitrans Medical Transport | Emergency Medical Service Facilities | Critical Community And Emergency Facilities | -82.0871 | 26.9759 | 2,287 | (0.1) |  | $102K | Minimal |
| 111 | Electrical/Mechanical Vault | Parks | Natural, Cultural, And Historical Resources | -82.1816 | 26.9984 | 624 | 0.6 | Affected | $100K | Minimal |
| 112 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1481 | 27.0046 | 3,478 | (0.1) |  | $92K | Minimal |
| 113 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1481 | 27.0049 | 1,528 | 0.2 | Affected | $71K | Minimal |
| 114 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1456 | 27.0063 | 863 | 0.7 | Affected | $71K | Minimal |
| 115 | Presbyterian Homes Of Charlotte* | Affordable Public Housing | Critical Community And Emergency Facilities | -82.1014 | 26.9908 | 3,414 | 0.1 | Affected | $62K | Minimal |
| 116 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities | -82.1112 | 26.9617 | 196 | 8.1 | Destroyed | $56K | Minimal |
| 117 | CCU Eastport Treatment Plant* | Wastewater Treatment Facilities | Critical Infrastructure | -82.0343 | 26.9842 | 1,206 | 0.7 | Affected | $53K | Minimal |
| 118 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1479 | 27.0049 | 1,545 | - | Affected | $45K | Minimal |
| 119 | Quesada Lift Station | Lift Stations | Critical Infrastructure | -82.1308 | 27.0110 | 993 | 0.8 | Affected | $41K | Minimal |
| 120 | P.C. Beach Complex* | Community Centers | Critical Community And Emergency Facilities | -82.1117 | 26.9617 | 137 | 8.4 | Destroyed | $40K | Minimal |
| 121 | Port Charlotte High School* | Schools | Critical Community And Emergency Facilities | -82.1490 | 27.0052 | 409 | 0.6 | Affected | $31K | Minimal |
| 122 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1478 | 27.0052 | 1,099 | - |  | $29K | Minimal |
| 123 | CCU Eastport Treatment Plant* | Wastewater Treatment Facilities | Critical Infrastructure | -82.0359 | 26.9832 | 684 | 0.5 | Affected | $29K | Minimal |
| 124 | Quesada Lift Station* | Lift Stations | Critical Infrastructure | -82.1306 | 27.0111 | 586 | 1.5 | Minor | $28K | Minimal |
| 125 | Gleneagles Water Booster Station* | Pump Stations | Critical Infrastructure | -82.0795 | 26.9892 | 405 | 3.6 | Major | $26K | Minimal |
| 126 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1478 | 27.0054 | 919 | (0.2) |  | $24K | Minimal |
| 127 | Eagle St Rwbs* | Pump Stations | Critical Infrastructure | -82.1114 | 27.0019 | 469 | 0.3 | Affected | $18K | Minimal |
| 128 | Charlotte Vocational Technical Center* | Schools | Critical Community And Emergency Facilities | -82.1477 | 27.0049 | 472 | - | Affected | $14K | Minimal |
| 129 | Mid County Mini Transfer Station* | Solid And Hazardous Waste Facilities | Critical Infrastructure | -82.1302 | 27.0193 | 6,231 | - |  | $11K | Minimal |
| 130 | Eagle St Rwbs* | Pump Stations | Critical Infrastructure | -82.1115 | 27.0018 | 243 | 0.7 | Affected | $10K | Minimal |
| 131 | Gleneagles Water Booster Station* | Pump Stations | Critical Infrastructure | -82.0796 | 26.9890 | 153 | 2.8 | Major | $9K | Minimal |

## Adaptation Focus Area 3 – West

### Focus Area Overview

Western Charlotte County features barrier-island-adjacent communities that face substantial coastal surge risk, king tide flooding, and long-term sea level change threats. The County’s VA inventory identifies multiple critical assets and residential concentrations in these regions that would require evacuation in severe storm scenarios. The area is also being considered for enhanced flood-monitoring systems, such as remote storm-elevation sensors along the Myakka River, Charlotte Harbor, and Lemon Bay, efforts aimed at improving preparedness and real-time response.

Area 3 west, while containing fewer total facility buildings (35 out of 256), plays a critical role in supporting essential services, especially in the domain of infrastructure. Area west has the highest number of wastewater treatment facilities (30 of 55) and pump stations (12 of 18), indicating its significance for water management and operational resilience. The area maintains a balanced mix of schools, fire stations, and community centers, ensuring continued educational access and community support despite its smaller size. Although Area west houses fewer health care and government facilities compared to Area mid, its infrastructure profile positions it as a vital support area for emergency response and service continuity, especially in scenarios involving water and waste management. This distribution reflects Area west’s strategic value in maintaining community resilience and operational stability during disruptive events.

### Impacted Facilities

presents a ranked list of facility buildings within Adaptation Focus Area 3– West, that are anticipated to be impacted by 100-year storm surge events. These assets have been identified based on projected flood depths, including schools, fire stations, local government offices, and health care centers. The preliminary ranking is decided by the estimated building (as described in Appendix D Technical Approach and Appendix E Exposure and Sensitivity Analysis). An asterisk next to an asset name indicates a 0.5 ft first floor elevation was assumed for estimating flood losses due to limited data. Loss estimates can be revised if more precise elevation information becomes available.

Table 11 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 3 - West under Current Condition

| Rank | Asset Name | Asset Type | Asset Class | Longitude | Latitude | Area (sqft) | Potential Flood Depth (ft) | Estimated Damage Severity | Estimated 100-year Storm Event Losses ($) | Estimated 100-year Storm Event Losses Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Myakka River Elementary School | Schools | Critical Community And Emergency Facilities | -82.2351 | 26.9412 | 109,481 | 1.2 | Minor | $44,230K | High |
| 2 | Lemon Bay High School | Schools | Critical Community And Emergency Facilities | -82.3326 | 26.9276 | 132,676 | 0.3 | Affected | $12,286K | Medium |
| 3 | Alterra Sterling House Of Englewood | Health Care Facilities | Critical Community And Emergency Facilities | -82.3025 | 26.8888 | 34,396 | 0.4 | Affected | $3,674K | Minimal |
| 4 | Charlotte County Fire & Ems St 10 | Fire Stations | Critical Community And Emergency Facilities | -82.3155 | 26.8682 | 2,157 | 8.7 | Destroyed | $3,559K | Minimal |
| 5 | Lemon Bay High School | Schools | Critical Community And Emergency Facilities | -82.3321 | 26.9269 | 132,707 | (0.3) |  | $3,473K | Minimal |
| 6 | Placida Post Office | Communications Facilities | Critical Infrastructure | -82.2926 | 26.8555 | 17,522 | 0.7 | Affected | $1,552K | Minimal |
| 7 | Englewood Fire Dept Station 76* | Fire Stations | Critical Community And Emergency Facilities | -82.2877 | 26.8520 | 1,814 | 5.1 | Major | $942K | Minimal |
| 8 | Englewood Fire Dept Station 76* | Fire Stations | Critical Community And Emergency Facilities | -82.2877 | 26.8517 | 4,687 | 1.2 | Minor | $521K | Minimal |
| 9 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2899 | 26.8717 | 10,461 | (0.1) |  | $408K | Minimal |
| 10 | Englewood Fd St 74 | Fire Stations | Critical Community And Emergency Facilities | -82.2900 | 26.9090 | 6,399 | (0.4) |  | $244K | Minimal |
| 11 | Five Lands Water Booster Station | Pump Stations | Critical Infrastructure | -82.2982 | 26.8760 | 2,463 | 2.3 | Major | $135K | Minimal |
| 12 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2891 | 26.8716 | 3,220 | (1.4) |  | $116K | Minimal |
| 13 | Five Lands Water Booster Station | Pump Stations | Critical Infrastructure | -82.2979 | 26.8761 | 2,107 | 2.2 | Major | $113K | Minimal |
| 14 | Englewood Fd St 72 | Fire Stations | Critical Community And Emergency Facilities | -82.3107 | 26.8981 | 2,886 | (2.4) |  | $97K | Minimal |
| 15 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2882 | 26.8718 | 2,217 | (1.1) |  | $81K | Minimal |
| 16 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2887 | 26.8712 | 2,190 | (1.4) |  | $79K | Minimal |
| 17 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2719 | 26.8926 | 1,416 | - | Affected | $53K | Minimal |
| 18 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2883 | 26.8714 | 1,436 | (1.2) |  | $52K | Minimal |
| 19 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2727 | 26.8924 | 1,569 | (0.2) |  | $49K | Minimal |
| 20 | CCU Westport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2040 | 26.9347 | 1,176 | 0.2 | Affected | $48K | Minimal |
| 21 | Lemon Bay High School | Schools | Critical Community And Emergency Facilities | -82.3318 | 26.9274 | 1,588 | (1.3) |  | $39K | Minimal |
| 22 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2896 | 26.8717 | 782 | (1.5) |  | $28K | Minimal |
| 23 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8719 | 727 | (1.2) |  | $27K | Minimal |
| 24 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2536 | 26.9894 | 671 | (0.8) |  | $25K | Minimal |
| 25 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8716 | 629 | (1.4) |  | $23K | Minimal |
| 26 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2539 | 26.9898 | 560 | (1.0) |  | $21K | Minimal |
| 27 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2881 | 26.8715 | 557 | (1.3) |  | $20K | Minimal |
| 28 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2724 | 26.8930 | 602 | (0.3) |  | $18K | Minimal |
| 29 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2731 | 26.8924 | 597 | (0.3) |  | $18K | Minimal |
| 30 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2896 | 26.8718 | 446 | (1.1) |  | $16K | Minimal |
| 31 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2885 | 26.8713 | 355 | (1.5) |  | $13K | Minimal |
| 32 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2885 | 26.8716 | 339 | (1.5) |  | $12K | Minimal |
| 33 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2731 | 26.8923 | 358 | (0.2) |  | $12K | Minimal |
| 34 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2730 | 26.8926 | 364 | (0.3) |  | $11K | Minimal |
| 35 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2882 | 26.8718 | 270 | (1.5) |  | $10K | Minimal |
| 36 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2534 | 26.9896 | 233 | (0.9) |  | $9K | Minimal |
| 37 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2888 | 26.8717 | 242 | (1.6) |  | $9K | Minimal |
| 38 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2540 | 26.9902 | 217 | (1.0) |  | $8K | Minimal |
| 39 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2884 | 26.8717 | 166 | (1.3) |  | $6K | Minimal |
| 40 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2537 | 26.9895 | 121 | (0.9) |  | $4K | Minimal |
| 41 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2727 | 26.8926 | 129 | (0.4) |  | $4K | Minimal |
| 42 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2887 | 26.8720 | 101 | (1.4) |  | $4K | Minimal |
| 43 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2728 | 26.8923 | 129 | (0.6) |  | $3K | Minimal |
| 44 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8717 | 45 | (1.5) |  | $2K | Minimal |
| 45 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8717 | 41 | (1.6) |  | $1K | Minimal |
| 46 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8717 | 38 | (1.6) |  | $1K | Minimal |

Table 12 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 3 - West when Change in Sea Level Exceeds 1 ft

| Rank | Asset Name | Asset Type | Asset Class | Longitude | Latitude | Area (sqft) | Potential Flood Depth (ft) | Estimated Damage Severity | Estimated 100-year Storm Event Losses ($) | Estimated 100-year Storm Event Losses Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Myakka River Elementary School | Schools | Critical Community And Emergency Facilities | -82.2351 | 26.9412 | 109,481 | 2.2 | Major | $45,592K | High |
| 2 | Lemon Bay High School | Schools | Critical Community And Emergency Facilities | -82.3326 | 26.9276 | 132,676 | 1.3 | Minor | $40,408K | High |
| 3 | Lemon Bay High School | Schools | Critical Community And Emergency Facilities | -82.3321 | 26.9269 | 132,707 | 0.7 | Affected | $11,315K | Medium |
| 4 | Alterra Sterling House Of Englewood | Health Care Facilities | Critical Community And Emergency Facilities | -82.3025 | 26.8888 | 34,396 | 1.4 | Minor | $8,293K | Low |
| 5 | Charlotte County Fire & Ems St 10 | Fire Stations | Critical Community And Emergency Facilities | -82.3155 | 26.8682 | 2,157 | 9.7 | Destroyed | $3,622K | Minimal |
| 6 | Placida Post Office | Communications Facilities | Critical Infrastructure | -82.2926 | 26.8555 | 17,522 | 1.8 | Minor | $2,523K | Minimal |
| 7 | Englewood Fire Dept Station 76* | Fire Stations | Critical Community And Emergency Facilities | -82.2877 | 26.8520 | 1,814 | 6.1 | Destroyed | $1,027K | Minimal |
| 8 | Englewood Fire Dept Station 76* | Fire Stations | Critical Community And Emergency Facilities | -82.2877 | 26.8517 | 4,687 | 2.2 | Major | $978K | Minimal |
| 9 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2899 | 26.8717 | 10,461 | 0.9 | Minor | $480K | Minimal |
| 10 | CCU Westport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2044 | 26.9351 | 8,749 | 2.0 | Minor | $472K | Minimal |
| 11 | Englewood Fd St 74 | Fire Stations | Critical Community And Emergency Facilities | -82.2900 | 26.9090 | 6,399 | 0.6 | Affected | $454K | Minimal |
| 12 | Five Lands Water Booster Station | Pump Stations | Critical Infrastructure | -82.2982 | 26.8760 | 2,463 | 3.4 | Major | $153K | Minimal |
| 13 | CCU Westport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2042 | 26.9347 | 2,863 | 1.3 | Minor | $141K | Minimal |
| 14 | Five Lands Water Booster Station | Pump Stations | Critical Infrastructure | -82.2979 | 26.8761 | 2,107 | 3.2 | Major | $130K | Minimal |
| 15 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2891 | 26.8716 | 3,220 | (0.4) |  | $123K | Minimal |
| 16 | Englewood Fd St 72 | Fire Stations | Critical Community And Emergency Facilities | -82.3107 | 26.8981 | 2,886 | (1.4) |  | $104K | Minimal |
| 17 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2882 | 26.8718 | 2,217 | (0.1) |  | $86K | Minimal |
| 18 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2887 | 26.8712 | 2,190 | (0.4) |  | $84K | Minimal |
| 19 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2537 | 26.9897 | 1,788 | - | Affected | $71K | Minimal |
| 20 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2727 | 26.8924 | 1,569 | 0.8 | Affected | $65K | Minimal |
| 21 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2719 | 26.8926 | 1,416 | 1.1 | Minor | $61K | Minimal |
| 22 | CCU Westport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2040 | 26.9347 | 1,176 | 1.2 | Minor | $57K | Minimal |
| 23 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2883 | 26.8714 | 1,436 | (0.2) |  | $56K | Minimal |
| 24 | Pump Stations* | Pump Stations | Critical Infrastructure | -82.3037 | 26.9353 | 981 | 1.4 | Minor | $45K | Minimal |
| 25 | Lemon Bay High School | Schools | Critical Community And Emergency Facilities | -82.3318 | 26.9274 | 1,588 | (0.2) |  | $42K | Minimal |
| 26 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2896 | 26.8717 | 782 | (0.5) |  | $30K | Minimal |
| 27 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8719 | 727 | (0.2) |  | $28K | Minimal |
| 28 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2536 | 26.9894 | 671 | 0.2 | Affected | $27K | Minimal |
| 29 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2724 | 26.8930 | 602 | 0.7 | Affected | $25K | Minimal |
| 30 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2731 | 26.8924 | 597 | 0.7 | Affected | $24K | Minimal |
| 31 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8716 | 629 | (0.4) |  | $24K | Minimal |
| 32 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2539 | 26.9898 | 560 | - |  | $22K | Minimal |
| 33 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2881 | 26.8715 | 557 | (0.3) |  | $21K | Minimal |
| 34 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2896 | 26.8718 | 446 | (0.1) |  | $17K | Minimal |
| 35 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2730 | 26.8926 | 364 | 0.8 | Affected | $15K | Minimal |
| 36 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2731 | 26.8923 | 358 | 0.8 | Affected | $15K | Minimal |
| 37 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2885 | 26.8713 | 355 | (0.5) |  | $13K | Minimal |
| 38 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2885 | 26.8716 | 339 | (0.5) |  | $13K | Minimal |
| 39 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2882 | 26.8718 | 270 | (0.5) |  | $10K | Minimal |
| 40 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2534 | 26.9896 | 233 | 0.1 | Affected | $9K | Minimal |
| 41 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2888 | 26.8717 | 242 | (0.6) |  | $9K | Minimal |
| 42 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2540 | 26.9902 | 217 | - | Affected | $9K | Minimal |
| 43 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2884 | 26.8717 | 166 | (0.3) |  | $6K | Minimal |
| 44 | Pump Stations* | Pump Stations | Critical Infrastructure | -82.3039 | 26.9352 | 119 | 2.1 | Major | $6K | Minimal |
| 45 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2727 | 26.8926 | 129 | 0.7 | Affected | $5K | Minimal |
| 46 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2728 | 26.8923 | 129 | 0.5 | Affected | $5K | Minimal |
| 47 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2537 | 26.9895 | 121 | 0.1 | Affected | $5K | Minimal |
| 48 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2887 | 26.8720 | 101 | (0.4) |  | $4K | Minimal |
| 49 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8717 | 45 | (0.5) |  | $2K | Minimal |
| 50 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8717 | 41 | (0.6) |  | $2K | Minimal |
| 51 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8717 | 38 | (0.6) |  | $1K | Minimal |

Table 13 Preliminary Ranking of Impacted Facilities in Adaptation Focus Area 3 - West when Change in Sea Level Exceeds 3 ft

| Rank | Asset Name | Asset Type | Asset Class | Longitude | Latitude | Area (sqft) | Potential Flood Depth (ft) | Estimated Damage Severity | Estimated 100-year Storm Event Losses ($) | Estimated 100-year Storm Event Losses Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Lemon Bay High School | Schools | Critical Community And Emergency Facilities | -82.3326 | 26.9276 | 132,676 | 3.2 | Major | $70,659K | High |
| 2 | Myakka River Elementary School | Schools | Critical Community And Emergency Facilities | -82.2351 | 26.9412 | 109,481 | 4.2 | Major | $49,010K | High |
| 3 | Lemon Bay High School | Schools | Critical Community And Emergency Facilities | -82.3321 | 26.9269 | 132,707 | 2.6 | Major | $23,406K | High |
| 4 | Alterra Sterling House Of Englewood | Health Care Facilities | Critical Community And Emergency Facilities | -82.3025 | 26.8888 | 34,396 | 3.4 | Major | $12,200K | Medium |
| 5 | Placida Post Office | Communications Facilities | Critical Infrastructure | -82.2926 | 26.8555 | 17,522 | 3.7 | Major | $3,995K | Minimal |
| 6 | Charlotte County Fire & Ems St 10 | Fire Stations | Critical Community And Emergency Facilities | -82.3155 | 26.8682 | 2,157 | 11.6 | Destroyed | $3,758K | Minimal |
| 7 | Vineland Elementary School* | Schools | Critical Community And Emergency Facilities | -82.2839 | 26.9141 | 92,992 | - | Affected | $2,537K | Minimal |
| 8 | Englewood Fire Dept Station 76* | Fire Stations | Critical Community And Emergency Facilities | -82.2877 | 26.8517 | 4,687 | 4.2 | Major | $2,094K | Minimal |
| 9 | Englewood Fd St 74 | Fire Stations | Critical Community And Emergency Facilities | -82.2900 | 26.9090 | 6,399 | 2.5 | Major | $1,613K | Minimal |
| 10 | Englewood Fire Dept Station 76* | Fire Stations | Critical Community And Emergency Facilities | -82.2877 | 26.8520 | 1,814 | 8.0 | Destroyed | $1,056K | Minimal |
| 11 | L. A. Ainger Middle School* | Schools | Critical Community And Emergency Facilities | -82.2875 | 26.9130 | 32,969 | (0.2) |  | $867K | Minimal |
| 12 | Englewood Fd St 75* | Fire Stations | Critical Community And Emergency Facilities | -82.2775 | 26.9353 | 6,210 | 1.5 | Minor | $863K | Minimal |
| 13 | L. A. Ainger Middle School* | Schools | Critical Community And Emergency Facilities | -82.2868 | 26.9130 | 28,481 | (0.2) |  | $752K | Minimal |
| 14 | L. A. Ainger Middle School* | Schools | Critical Community And Emergency Facilities | -82.2875 | 26.9140 | 27,434 | (0.3) |  | $719K | Minimal |
| 15 | CCU Westport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2044 | 26.9351 | 8,749 | 3.9 | Major | $710K | Minimal |
| 16 | L. A. Ainger Middle School* | Schools | Critical Community And Emergency Facilities | -82.2878 | 26.9123 | 24,334 | - |  | $648K | Minimal |
| 17 | L. A. Ainger Middle School* | Schools | Critical Community And Emergency Facilities | -82.2862 | 26.9135 | 22,683 | (0.3) |  | $593K | Minimal |
| 18 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2899 | 26.8717 | 10,461 | 2.9 | Major | $593K | Minimal |
| 19 | Englewood Charlotte Library | Disaster Recovery Centers, Community Centers | Critical Community And Emergency Facilities | -82.2929 | 26.9351 | 18,002 | (1.2) |  | $448K | Minimal |
| 20 | Tidewell Hospice Inc | Health Care Facilities | Critical Community And Emergency Facilities | -82.2552 | 26.9333 | 8,238 | (0.3) |  | $354K | Minimal |
| 21 | L. A. Ainger Middle School* | Schools | Critical Community And Emergency Facilities | -82.2875 | 26.9135 | 12,391 | (0.3) |  | $324K | Minimal |
| 22 | Charlotte County Fire & Ems St 4* | Fire Stations | Critical Community And Emergency Facilities | -82.2231 | 26.9173 | 4,261 | 0.4 | Affected | $255K | Minimal |
| 23 | L. A. Ainger Middle School* | Schools | Critical Community And Emergency Facilities | -82.2879 | 26.9130 | 8,846 | (0.1) |  | $235K | Minimal |
| 24 | Lemon Bay High School | Schools | Critical Community And Emergency Facilities | -82.3318 | 26.9274 | 1,588 | 1.7 | Minor | $216K | Minimal |
| 25 | Englewood Fd St 72 | Fire Stations | Critical Community And Emergency Facilities | -82.3107 | 26.8981 | 2,886 | 0.6 | Affected | $204K | Minimal |
| 26 | L. A. Ainger Middle School* | Schools | Critical Community And Emergency Facilities | -82.2869 | 26.9139 | 7,364 | (0.2) |  | $195K | Minimal |
| 27 | Five Lands Water Booster Station | Pump Stations | Critical Infrastructure | -82.2982 | 26.8760 | 2,463 | 5.3 | Major | $191K | Minimal |
| 28 | CCU Westport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2042 | 26.9347 | 2,863 | 3.3 | Major | $187K | Minimal |
| 29 | L. A. Ainger Middle School* | Schools | Critical Community And Emergency Facilities | -82.2879 | 26.9135 | 6,655 | (0.2) |  | $175K | Minimal |
| 30 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2891 | 26.8716 | 3,220 | 1.6 | Minor | $164K | Minimal |
| 31 | Five Lands Water Booster Station | Pump Stations | Critical Infrastructure | -82.2979 | 26.8761 | 2,107 | 5.2 | Major | $160K | Minimal |
| 32 | L. A. Ainger Middle School* | Schools | Critical Community And Emergency Facilities | -82.2869 | 26.9135 | 5,924 | (0.4) |  | $154K | Minimal |
| 33 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2882 | 26.8718 | 2,217 | 1.9 | Minor | $118K | Minimal |
| 34 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2887 | 26.8712 | 2,190 | 1.6 | Minor | $112K | Minimal |
| 35 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2537 | 26.9897 | 1,788 | 2.0 | Minor | $97K | Minimal |
| 36 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2727 | 26.8924 | 1,569 | 2.8 | Major | $92K | Minimal |
| 37 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2719 | 26.8926 | 1,416 | 3.0 | Major | $86K | Minimal |
| 38 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2883 | 26.8714 | 1,436 | 1.8 | Minor | $76K | Minimal |
| 39 | CCU Westport Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2040 | 26.9347 | 1,176 | 3.2 | Major | $72K | Minimal |
| 40 | Pump Stations* | Pump Stations | Critical Infrastructure | -82.3037 | 26.9353 | 981 | 3.4 | Major | $61K | Minimal |
| 41 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2896 | 26.8717 | 782 | 1.5 | Minor | $39K | Minimal |
| 42 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8719 | 727 | 1.8 | Minor | $38K | Minimal |
| 43 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2536 | 26.9894 | 671 | 2.2 | Major | $37K | Minimal |
| 44 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2724 | 26.8930 | 602 | 2.7 | Major | $35K | Minimal |
| 45 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2731 | 26.8924 | 597 | 2.7 | Major | $34K | Minimal |
| 46 | L. A. Ainger Middle School* | Schools | Critical Community And Emergency Facilities | -82.2871 | 26.9125 | 1,271 | (0.4) |  | $33K | Minimal |
| 47 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8716 | 629 | 1.6 | Minor | $32K | Minimal |
| 48 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2539 | 26.9898 | 560 | 2.0 | Minor | $30K | Minimal |
| 49 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2881 | 26.8715 | 557 | 1.7 | Minor | $29K | Minimal |
| 50 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2896 | 26.8718 | 446 | 1.9 | Minor | $24K | Minimal |
| 51 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2730 | 26.8926 | 364 | 2.7 | Major | $21K | Minimal |
| 52 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2731 | 26.8923 | 358 | 2.8 | Major | $21K | Minimal |
| 53 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2885 | 26.8713 | 355 | 1.4 | Minor | $18K | Minimal |
| 54 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2885 | 26.8716 | 339 | 1.5 | Minor | $17K | Minimal |
| 55 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2882 | 26.8718 | 270 | 1.5 | Minor | $14K | Minimal |
| 56 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2534 | 26.9896 | 233 | 2.1 | Major | $13K | Minimal |
| 57 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2888 | 26.8717 | 242 | 1.4 | Minor | $12K | Minimal |
| 58 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2540 | 26.9902 | 217 | 2.0 | Minor | $12K | Minimal |
| 59 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2884 | 26.8717 | 166 | 1.7 | Minor | $9K | Minimal |
| 60 | Pump Stations* | Pump Stations | Critical Infrastructure | -82.3039 | 26.9352 | 119 | 4.1 | Major | $8K | Minimal |
| 61 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2727 | 26.8926 | 129 | 2.6 | Major | $7K | Minimal |
| 62 | CCU - Booster Station 6 | Pump Stations | Critical Infrastructure | -82.2728 | 26.8923 | 129 | 2.4 | Major | $7K | Minimal |
| 63 | West County Mini Transfer Station | Solid And Hazardous Waste Facilities | Critical Infrastructure | -82.2362 | 26.9279 | 4,190 | (0.1) |  | $7K | Minimal |
| 64 | CCU Gulf Cove Water Treatment Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2537 | 26.9895 | 121 | 2.1 | Minor | $7K | Minimal |
| 65 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2887 | 26.8720 | 101 | 1.6 | Minor | $5K | Minimal |
| 66 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8717 | 45 | 1.5 | Minor | $2K | Minimal |
| 67 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8717 | 41 | 1.4 | Minor | $2K | Minimal |
| 68 | Rotonda Reserve Osmosis Plant | Wastewater Treatment Facilities | Critical Infrastructure | -82.2889 | 26.8717 | 38 | 1.3 | Minor | $2K | Minimal |

# Next Steps

The vulnerability assessment indicates that several facility types, including pump stations, fire stations, healthcare facilities, and community facilities, experience higher levels of disruption under storm surge flooding scenarios. Facilities identified and ranked through this effort are anticipated to be further evaluated and prioritized in future phases based on detailed vulnerability, feasibility, cost, and benefit considerations.

The subsequent section details the specific areas of alignment with the strategies outlined in the Charlotte County Local Mitigation Strategy, demonstrating the intended interlinked initiatives undertaken by local agencies to strengthen community resilience. Additionally, recommended actions for addressing facility vulnerabilities are identified, alongside a summary of potential funding sources available to support these resilience-building measures.

## Alignment with Charlotte County Local Mitigation Strategy

This section examines how vulnerable facilities identified in this assessment correspond to mitigation projects listed in the Charlotte County Local Mitigation Strategy (LMS). The comparison focuses on whether existing/proposed mitigation projects address critical facilities and infrastructure identified as vulnerable in the vulnerability assessment. The results show that several ongoing or planned projects support the resilience of key emergency and infrastructure systems within the county. Selected examples of aligned mitigation projects that address identified facility vulnerabilities are summarized in Table 14.

Table 14 Alignment between Identified Vulnerable Facilities and LMS Projects

| Projects | Hazards Mitigated | Agency Responsible for Implementation | Status | Timeframe to Complete |
| --- | --- | --- | --- | --- |
| Hardening of Westport and Rotonda Wastewater Treatment Plants | Hurricanes, Tropical storms, Flooding | Charlotte County Utilities | Submitted under DR-4806 | 36 months from award date |
| Charlotte County Lift Station Portable Generator Resilience Project | Hurricanes, Tropical storms, Flooding | Charlotte County Utilities | Submitted under DR-4806 | 36 months from award date |
| Punta Gorda Library Mobile-Generator | Hurricanes, Tropical storms, Flooding | Charlotte County Parks | Submitted under DR-4828 | 36 months from award date |
| Punta Gorda Library Floodproofing | Hurricanes, Tropical storms, Flooding | Charlotte County Parks | Submitted under DR-4834 | 36 months from award date |
| Charlotte Harbor Event Center Floodproofing | Hurricanes, Tropical storms, Flooding | Charlotte County Parks | Submitted under DR-4834 | 36 months from award date |

## Recommended Actions to Increase Resilience of Charlotte County

This section outlines recommended actions to enhance resilience against storm-surge events.

### Countywide Planning and Policy

Based on the vulnerability assessment results, the following policy and planning actions are recommended to reduce long-term storm-surge flood risks in Charlotte County.

#### Risk Assessment and Data Integration

- Expand the analysis completed by this study to include private property, social, and economic concerns to develop a full inventory of risk from collective stressors.

- Integrate climate hazard projections (e.g., sea level change, extreme rainfall, and storm surge) into future land-use, infrastructure investment decisions, and capital improvement programs.

#### Floodplain Management and Development Policy

- Consider strengthening the existing Charlotte County floodplain ordinance by increasing freeboard requirements above the 100-year flood elevation (potentially to the 500-year flood) to further reduce future flood risks.

- Establish a minimal roadway level of service or accessibility standards for transportation infrastructure located in flood-prone areas to ensure continued access during storm surge events.

- Encourage elevation or floodproofing retrofits for existing structures located within high-risk flood zones identified in the vulnerability assessment.

#### Nature-based Solutions and Risk Reduction Strategies

- Promote nature-based solutions, such as wetlands restoration and green stormwater infrastructure, to improve flood storage and water quality.

- Evaluate voluntary property acquisition and demolition programs for repetitive flood loss areas. These programs purchase high-risk properties, remove structures, and convert the land into open space or natural buffers, which can permanently reduce flood risks.

- Undertake an effort to define county policies on managed retreat from future inundated areas.

#### Funding and Implementation Coordination

- Complete a detailed scan of the funding sources that may be available to Charlotte County to address resilience concerns outlined in this assessment.

- Develop grant/formula funding requests for various eligible sources (IIJA, FEMA, FDEP, etc.).

- Strengthen coordination between county departments to integrate resilience considerations into transportation, utilities, and capital improvement planning.

#### Public Awareness and Community Engagement

- Conduct an educational and public information campaign to increase stakeholder awareness of the concerns, and planned county actions to address them.

### Wastewater Treatment Facility

Pump stations and lift stations are particularly vulnerable to flooding due to their low elevations and reliance on electrical and mechanical equipment. Recommended adaptation measures include:

- Evaluate the elevation of critical wastewater treatment equipment and consider elevating or floodproofing vulnerable components.

- Improve redundancy in power supply systems, such as installing backup generators and ensuring adequate fuel storage for extended outages.

- Upgrade drainage and stormwater management around wastewater facilities to prevent onsite flooding during extreme rainfall events.

- Develop emergency response procedures to maintain wastewater service continuity during hurricanes and major storm events.

- Consider alternative or supplemental power technologies for lift stations, such as solar photovoltaic systems with battery storage, to provide additional power resilience during grid outages.

- Evaluate the feasibility of microgrid systems or hybrid energy systems combining solar power, battery storage, and backup generators to improve redundancy and reliability for critical wastewater infrastructure.

### Emergency Response Facilities (Fire Stations)

Fire stations are critical for emergency response during disasters and must remain operational during flooding events. Recommended strategies include:

- Elevate critical building systems, including electrical panels, communications equipment, and generators above projected flood levels.

- Install deployable flood barriers or floodproof doors to reduce water intrusion.

- Elevate emergency vehicles or relocate vehicle storage areas where flooding risk is significant.

- Ensure backup power and communication systems remain functional during prolonged flood events.

### Healthcare Facilities and Hospitals

Healthcare facilities must maintain continuous operation during extreme events. Flood resilience strategies include:

- Elevate or floodproof mechanical, electrical, and medical support systems vulnerable to storm surge flooding.

- Ensure backup power systems and fuel storage are protected from floodwaters.

- Protect building access points and entrances using flood barriers or elevation improvements.

- Develop contingency plans to maintain critical medical services during flood disruptions.

### Schools and Shelter Facilities

Schools represent a large share of public facilities and may serve as emergency shelters during disasters. Recommended measures include:

- Elevate critical infrastructure such as electrical systems, HVAC equipment, and generators above projected flood levels.

- Improve stormwater drainage systems around school campuses to reduce localized flooding during surge events.

- Retrofit selected schools to function as flood-resilient emergency shelters.

- Protect building entrances and utility rooms using flood barriers or floodproofing measures.

### Community Facilities

Community centers and other public facilities often serve as gathering points during emergencies and may experience flood-related disruptions. Recommended measures include:

- Elevate or floodproof critical mechanical and electrical equipment within facilities.

- Improve site drainage and install localized flood protection where feasible.

- Retrofit community facilities to support emergency operations during storm surge events.

### Local Government and Administrative Facilities

Local government facilities support emergency coordination and recovery operations. Recommended measures include:

- Protect critical information technology infrastructure from flood exposure.

- Elevate electrical systems and communications equipment above flood-prone levels.

- Ensure backup power systems are protected from storm surge flooding.

### Parks

Parks serve as vital spaces for recreation, community gatherings, and environmental stewardship, making their resilience essential during emergencies. Recommended measures include:

- Revise and enhance the Parks Master Plan to address evolving needs and resilience strategies.

- Establish oyster reef-based living shorelines in coastal parks to improve natural defences and adaptation.

- Plant salt-resistant grasses and vegetation to withstand flooding and saline conditions.

- Develop shaded pedestrian zones to provide comfortable areas for visitors to escape heat.

- Allocate resources for temporary flood barriers to safeguard key park facilities as needed.

- Upgrade stormwater systems and apply leading management practices, such as installing upstream detention basins, to reduce flood risks.

### Transportation Infrastructure

Transportation infrastructure serves as the backbone of community mobility and economic activity, enabling the safe and efficient movement of people, goods, and emergency services across the region. Recommended measures include:

- Incorporate green infrastructure strategies such as bioswales, permeable surfaces, and natural drainage features near bridge approaches and transportation corridors to reduce localized flooding during storm surge events.

- Integrate flood-resilient design considerations into transportation infrastructure upgrades, including bridge retrofits and roadway improvements located in surge-prone areas.

### Homeowner Flood Resilience Actions

While the current vulnerability assessment primarily focuses on critical facilities and public infrastructure, expanding the analysis to include private residential properties in high-risk storm surge areas could provide additional opportunities to reduce long-term community flood risk. Recommendation actions include:

- Encourage property-level flood mitigation measures such as elevating utilities, installing flood barriers, or floodproofing critical building components.

- Develop partnerships with community organizations to promote residential resilience and access to mitigation funding programs.

- Provide education and outreach programs to inform homeowners about flood risk and available mitigation strategies.

## Industry Standards Cost Estimation

The purpose of providing industry-standard cost estimates for resilience upgrades and mitigation measures is to offer a preliminary reference for budgeting and planning. These estimates are intended to guide stakeholders in understanding potential financial requirements before undertaking detailed facility-level analyses. However, it is important to note that such figures are broad benchmarks only; actual costs may differ significantly depending on the project scale, location, facility age, current condition, and site-specific vulnerabilities. Published guidelines, historical benchmarks, and typical wage rates, equipment pricing guides, and material cost references are used to inform these estimates, with adjustments made to reflect regional context where appropriate. Despite these adjustments, the estimates do not reflect the unique specifics of Charlotte County and should not be relied upon for definitive budgeting. Labor, equipment, and material pricing are based on generalized rates and market data, and while regional conditions are considered, a detailed analysis for Charlotte County has not been conducted. For precise budgeting and implementation, a thorough facility-level assessment is required to address the unique needs of each asset and ensure aCCUracy.

Table 15 Industry Standard Costs Reference

| Recommended Actions | Note | Unit | Estimated Unit Cost (2025$) |
| --- | --- | --- | --- |
| Temporary Flood Barriers & Deployable Floodwalls |  | Square foot | $372.79 |
| Floodwalls |  | Square foot | $70.62 |
| Asset Elevation | Up to 1 ft | Each | $3,529 |
| Asset Elevation | 1-6 ft | Each | $6,072 |
| Dry Flood Proofing | Seal Building Wall | Square foot | $0.7 |
|  | Permanently Seal Building Openings | Each | $8,048 |
|  | Temporarily Flood Shields for Openings | Each | $4,158 |
|  | Building Interior Drainage | Each | $7,739 |
| Sandbag and sandbag alternatives |  | Linear foot | $27.02 |
| Detention ponds |  | Acre | $109,760.18 |
| Permeable pavers |  | Square foot | $40.96 |

## Potential Funding Sources

Identifying and securing potential funding sources is a critical step in advancing resilience initiatives for county infrastructure and emergency facilities. Access to federal and state programs can significantly bolster the county’s capacity to implement mitigation measures and respond effectively to climate-related hazards. By leveraging these funding opportunities, local authorities can ensure that vital assets remain operational during emergencies, while also fostering long-term community safety and sustainability.

### Potential Federal and State Funding Sources

Table 16 lists potential federal and state funding sources that can be accessed to support county resilience projects. By utilizing these resources, local authorities can strengthen the operational reliability of vital assets during emergencies and promote long-term community safety and sustainability.

Table 16 Potential Federal and State Funding Sources Funding Sources

| Funding Source | Level | Notes |
| --- | --- | --- |
| FEMA Building Resilient Infrastructure and Communities (BRIC) | Federal | The program aims to support communities as they build capability and capacity. BRIC also encourages and aids innovation. It helps partnerships grow; supports infrastructure projects; and fosters flexibility and consistency. |
| FEMA Hazard Mitigation Grant Program (HMGP) | Federal | Available following federal disaster declarations to support mitigation projects that reduce future disaster losses. To receive HMGP funding, all state, local, tribal and territorial governments must develop and adopt hazard mitigation plans. |
| HUD Community Development Block Grant - Mitigation (CDBG-MIT) | Federal | CDBG Mitigation (CDBG-MIT) is a unique and significant opportunity for eligible grantees to use this assistance in areas impacted by recent disasters to carry out strategic and high-impact activities to mitigate disaster risks and reduce future losses. |
| USDOT Promoting Resilient Operations for Transformative, Efficient, and Cost-Saving Transportation (PROTECT) | Federal | PROTECT provides funding to ensure surface transportation resilience to natural hazards including climate change, sea level change, flooding, extreme weather events, and other natural disasters through support of planning activities, resilience improvements, community resilience and evacuation routes, and at-risk coastal infrastructure. |
| NOAA Coastal Resilience Grants | Federal | The National Coastal Resilience Fund is a partnership between the National Fish and Wildlife Foundation and NOAA to enhance fish and wildlife habitat and protect coastal communities. The focus for these projects is on restoring, increasing, and strengthening natural infrastructure to protect communities while also enhancing habitats for fish and wildlife. |
| U.S. Fish and Wildlife Service Coastal Program | Federal | The program often supports ecosystem-based solutions that provide both environmental and flood resilience benefits. |
| NFWF National Coastal Resilience Fund | Federal | NFWF will make investments in planning, design, and implementation of nature-based solutions to enhance protection for coastal communities from the impacts of storms, floods, and other natural hazards while improving habitats for fish and wildlife. |
| Better Utilizing Investments to Leverage Development (BUILD) Grant Program ¦ US Department of Transportation | Federal | The BUILD grant program provides grants for surface transportation infrastructure projects with significant local or regional impact. Project sponsors, including state and local governments, counties, Tribal governments, transit agencies, and port authorities, can pursue multi-modal and multi-jurisdictional projects that are more difficult to fund through other grant programs. |
| USDOT Reconnecting Communities Pilot Program | Federal | Reconnecting Communities Pilot Program will restore community connectivity by removing, retrofitting, or mitigating highways or other transportation facilities that create barriers to community connectivity, including to mobility, access, or economic development. |
| DOI Refuge System Resiliency | Federal | To fund projects on National Wildlife Refuges and state wildlife management areas that combat invasive species, restore and increase the resiliency of habitats, and/or build resilient infrastructure, with a focus on nature-based solutions where possible. |
| USDA Watershed and Flood Prevention Operations | Federal | Provides planning, design and construction of measures that address resource concerns in a watershed. |
| USDA State Fire Assistance | Federal | Through the State Fire Assistance program, the Forest Service supports and assists State Foresters and local communities in building capacity for wildfire prevention, mitigation, control, and suppression on non-Federal lands. The program helps State agencies create more fire-adapted communities by implementing pre-fire prevention and mitigation programs and emphasizing pre-fire planning and risk reduction in the Wildland Urban Interface. The program funds important training in safer initial attack responses to wildfire that are also effective. Additionally, the program improves capacity to assist other Federal, State, and local agencies in aiding communities affected by fire and non-fire emergencies, such as hurricanes and floods. |
| FEMA Emergency Management Performance Grant (EMPG) | Federal | The Emergency Management Performance Grant provides state, local, tribal and territorial emergency management agencies with the resources required for implementation of the National Preparedness System. The EMPG’s allowable costs support efforts to build and sustain core capabilities across the prevention, protection, mitigation, response and recovery mission areas. |
| FEMA Safeguarding Tomorrow Revolving Loan Fund Program (STORM) | Federal | Provides capitalization grants to states, eligible federally recognized tribes, territories and the District of Columbia to establish revolving loan funds that provide hazard mitigation assistance for local governments to reduce risks from natural hazards and disasters. |
| FEMA Flood Mitigation Assistance Grants (National Flood Insurance Act Sec 1366) | Federal | The Flood Mitigation Assistance program makes Federal funds available to States, U.S. territories, Federally recognized Tribal governments, and local communities to reduce or eliminate the risk of repetitive flood damage to buildings and structures insured by the National Flood Insurance Program (NFIP), and with NFIP-participating communities. |
| USDA Emergency Community Water Assistance Grants (ECWAG) | Federal | This program helps eligible communities prepare, or recover from, an emergency that threatens the availability of safe, reliable drinking water. Events that can qualify as an emergency include: drought or flood, earthquake, tornado or hurricane, disease outbreak, chemical spill, leak or seepage, other disasters. |
| EPA Water Infrastructure Finance and Innovation Program | Federal | The WIFIA program accelerates investment in water and wastewater infrastructure of national and regional significance by offering creditworthy borrowers loans for up to 49 percent of eligible project costs. WIFIA can fund planning, design, and construction activities for a wide variety of eligible water infrastructure projects. |
| Florida Resilient Coastlines Program (FDEP) | State | The Resilient Florida Program includes a selection of grants that are available to counties, municipalities, special districts with specific responsibilities and regional resilience entities. To effectively address the impacts of flooding and sea level change that the state faces, eligible applicants may receive funding assistance to analyze and plan for vulnerabilities, as well as implement projects for adaptation and mitigation. |
| Florida Fish and Wildlife Conservation Commission (FWC) Grants | State | Supports environmental restoration efforts that can also reduce coastal flood risks. |

### Alternative Funding Sources

Charlotte County faces a likely public funding gap for future resilience investments. To help address this, we recommend developing a Resilience Funding and Financing Blueprint tailored to the County's key investment needs.

The blueprint development would involve three workstreams. First, a focused review of Charlotte County's resilience funding landscape, evaluating and prioritizing the most promising federal, state, and local funding opportunities. Second, an assessment of local revenue sources best suited to resilience expenditures — such as project revenues, performance-based incentives, taxes, charges, and tradable credits. Third, once specific projects are identified, the development of optimal funding and financing structures, including capital stacks aligned to project types, priorities, and capex/OpEx needs — drawing on instruments such as long-term debt, bonds, public-private partnerships, and revolving loan funds where appropriate.

The resulting blueprint would provide decision-making frameworks for revenue and financing, as well as an implementation guide for deploying these mechanisms across different resilience investment types over the short, medium, and long term.

# Summary

This document represents an intermediate step in the County’s resilience planning process. The identification and ranking of facilities within Adaptation Focus Areas establishes a foundation for future, more detailed analyses, including facility-level vulnerability assessments and project development efforts. Upon completion of this task, the ranked facilities and focus areas are anticipated to inform subsequent phases of work, including the prioritization of assets for detailed evaluation, concept-level adaptation strategy development, and advancement of implementation-ready projects. These next steps are expected to be pursued through future funding opportunities, such as the Resilient Florida Grant Program, allowing the County to transition from planning-level identification to targeted resilience investments.

Asset data for historical and cultural assets was sourced from the Bureau of Archaeological Research and consists of layers for cemeteries, historic bridges, historic structures, and resource groups (historical districts, archaeological districts or building complexes). These data are ultimately sourced from the Florida Master Site File (FMSF), the State of Florida's official inventory of historical and cultural Resources

This is a smaller structure associated with the Grace Street Health Center. The official name needs to be further determined.

This is a smaller structure associated with the Grace Street Health Center. The official name needs to be further determined.

This is a smaller structure associated with the Grace Street Health Center. The official name needs to be further determined.